3GPP TSG-SA2 Meeting #133	S2-1906131

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019		revision S2-1904937, 6043

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1283 | rev | 2 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction between MICO mode with active time and eDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to further optimize the power consumption during the active time for MICO mode is running, the UE may also request to use eDRX, same as PSM with active time and eDRX in LTE as specified in subclause 4.5.13.1 of TS23.682 and subclause 5.3.13 of TS24.301. When the UE requests the use of both MICO mode with active time and eDRX during a registration procedure, it is up to the network to decide to enable none, one of them or both. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Interaction between active time for MICO mode and eDRX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No guidance on network behavoir when the UE requests MICO mode with active time and eDRX during a registration procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | REV1 Removed paragraph of text Turned bullet list into sentence REV2 Changed local configuration into longer list of examples based on text in section 5.31.7.4 #### 5.31.7.4 MICO mode with Active Time During a Registration procedure the UE may optionally request an Active Time value from the AMF as part of MICO Mode negotiation. In response, if the AMF receives an Active Time value from the UE and determines that the MICO mode is allowed for the UE, the AMF may assign an Active Time value for the UE, e.g. based on local configuration, Expected UE Behaviour if available, UE requested Active Time value, UE subscription information and network policies |  |  |  |  |  |  |  |  |

##### ***** First change *****

#### 5.31.7.1	General

To enable UE power saving and to enhance MT reachability while using MICO mode, e.g. for CIoT, the following features are specified in the following clauses:

-	Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE; and

-	MICO mode with Extended Connected Time; and

-	MICO mode with Active Time; and

-	MICO mode and Periodic Registration Timer Control.

If a UE requests via NAS both to enable MICO mode with Active Time and extended idle mode DRX, e.g. based on local configuration, Expected UE Behaviour if available, UE requested Active Time value, UE subscription information and network policies etc, the AMF may decide to enable either MICO mode with or without Active Time or extended idle mode DRX or do both.

##### ***** Next change *****