3GPP TSG-SA WG2 Meeting #133	S2-1906566

# May 13 – 17, 2019 ; Reno, Nevada, USA					           (was S2-1905434)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1230 | rev | 6 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dedicated SMF selection for a 5G LAN group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  |  | Date: |  | 2019-04-29 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an EN for “Whether there are impacts to SMF selection to select this dedicated SMF is FFS.” This CR is trying to resolve this EN. The dedicated SMF set shall be selected for all the PDU sessions belongs to one 5G LAN group. The S-NSSAI and DNN shall be used for NRF to select SMF. Considering one 5G VN group is associated with one DNN, it is suggested the NRF can be locally configured to only select the dedicated SMF set based on the DNN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A dedicated SMF set is responsible for all the PDU Sessions for communication of a certain 5G-LAN group. Within this SMF set a dedicated SMF is responsible of managing the resources (e.g. N19 interfaces between UPF(s) serving the 5G VN group) that are common for all PDU Sessions to that 5G VN group. NOTE1: The network is configured so that the same SMF set is always selected for a certain 5G VN group |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to select the dedicated SMF for one 5G VN group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Change wrt to rev4: |  |  |  |  |  |  |  |  |

***** 1st Change *****

### 5.29.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G-LAN group

-	A dedicated SMF set is responsible for all the PDU Sessions for communication of a certain 5G-LAN group.

NOTE:	Having a dedicated SMF serving a 5G-LAN group does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G-LAN group

Editor's note:	The above bullet is FFS.

-	The UE provides a DNN associated with the 5G-LAN group to access the 5G LAN-type services for that 5G-LAN group, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G-LAN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G-LAN group. There is no 5G-LAN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G-LAN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G-LAN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G-LAN group.

***** End of Changes *****