3GPP TSG-SA WG2 Meeting #133	S2-1906575

# RENO, NEVADA, USA, MAY 13 – 17, 2019	(revision of S2-1905684)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1429 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of restrictions on 5G VN group communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Each particular 5G LAN-type service (i.e. a 5G VN) may have an allowed area. The 5GS shall configure and enforce restrictions on 5G VN group communication, e.g. based on location of group member, for a particular 5G VN. When UE is the member of a 5G VN group with an allowed area, reuse the LADN mechanism to enforce the restrictions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of restrictions on 5G VN group communication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for 5G LAN-type service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.2, 5.29.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.29	Support for 5G LAN-type service

5.29.2	5G LAN group management

5G System supports 5G LAN Group Management can be configured by a network administrator or can be managed dynamically by AF.

The 5G LAN group member is uniquely identifed by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [3] is applicable to 5G LAN-type services.

In order to support dynamic 5GLAN Group Management, the NEF exposes a set of service to manage (e.g. add/delete) 5G LAN group and 5G LAN member.

The 5G LAN group configuration is either provided by OA&M or provided by an AF to the NEF. The 5G LAN group configuration is stored in the UDR.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies. The 5G LAN group configuration in UDR may include the following parameters: GPSI (i.e. UE Identities of the 5G LAN group), PDU session type, DNN, S-NSSAI and VN Service Area.

The VN Service Area defines the area within which the group communication is allowed for the group member. If a VN Service Area is configured for a 5G VN, the 5GS considers the DNN associated with the 5G VN as a LADN DNN and enforces the 5G VN service area restriction as described in 5.29.5.

The third party AF may update the UE Identities of the 5G LAN group at any time after the initial provisioning.

Editor's note:	Relation between DNN and 5G LAN group, e.g. whether and how a 1:1 and/or 1:N relation is supported, is FFS.

The PCF delivers 5G LAN group configuration information (DNN, S-NSSAI, PDU session type) to the UE for each GPSI that belongs to a 5G-LAN group. The 5G LAN group configuration information is delivered in the URSP from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3 and TS 23.503 [45] clause 6.1.2.2.

5.29.5	Area Restriction on 5G LAN-type service

The 5G LAN-type service may be restricted to a certain service area as defined by the VN Service Area in the 5G VN group configuration data.

If a VN Service Area is configured for a 5G VN, the network considers the DNN associated with the 5G VN as a LADN DNN and should provide the related LADN Information, which includes the DNN associated with the 5G VN as a LADN DNN and the intersection of VN Service Area and UE’s registration area as the LAND Service Area, to the UE during Registration or UE Configuration Update procedures. The UE and the network follow clause 5.6.5 to enforce the area restriction for the 5G VN group communication.

**** End of Changes *****