SA WG2 Meeting #133 	S2-1906662

13 May -  17 May 2019, Reno, NV, USA

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1382 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Survival time EN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 5/16/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 23.501 section 5.27.2 TSC Assistance Information (TSCAI) the Editor's note: - Editor's note: Need for other parameters (e.g. survival time) FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IEEE and IEC are working together to define a joint IEC/IEEE 60802 TSN Profile for Industrial Automation [1]. This is a joint standardization work of IEC SC65C/MT9 and IEEE 802 to define TSN profiles for industrial automation. The standard defines time-sensitive networking profiles for industrial automation. The profiles select features, options, configurations, defaults, protocols, and procedures of bridges, end stations, and LANs to build industrial automation networks. There are two inputs to the IEC/IEEE 60802 TSN Profile for Industrial Automation specification, one addresses the industrial use cases [2], the other addresses the industrial requirement [3]. These two documents have been accepted enough to move on and work on the standard specification itself. "Survival Time" has not been considered in either of those documents. Moreover, Survival Time has not been defined as a parameter of any of the managed objects specified by IEEE 802.1Qcc [4] for TSN. I.E. CUC/CNC doesn’t provide survival time by IEEE 802.1Qcc definition. It is therefore proposed that: There is no need to consider “survival time” as TSCAI for 5G system. [1] IEEE https://1.ieee802.org/tsn/iec-ieee-60802/ [2] IEC/IEEE 60802 Industrial Requirements http://www.ieee802.org/1/files/public/docs2018/60802-industrial-use-cases-0918-v13.pdf [3] IEC/IEEE 60802 Industrial Requirements http://www.ieee802.org/1/files/public/docs2018/60802-industrial-requirements-1218-v12.pdf [4] IEEE 802.1Qcc 2018 - IEEE Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment 31: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements. https://standards.ieee.org/standard/802_1Qcc-2018.html SA1 has included the use of Survival Time to be further studied in Release 17(S1-191299) in TR 22.832 "Study on enhancements for cyber-physical control applications in vertical domains". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | “survival time” has no consequence on completion of Vertical_LAN work so unnecessary delays to complete the work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.27.2	TSC Assistance Information (TSCAI)

TSC assistance information describes TSC traffic characteristics for use in the 5G System. The knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. TSC assistance information, as defined in Table 5.27.2-1, is provided from SMF to 5G-AN, e.g. upon QoS flow establishment. The TSCAI parameters are set according to corresponding parameters obtained from the AF. The maximum value of TSC Burst Size should be mapped to a 5QI with MDBV that is equal or higher. For TSC QoS flows, MDBV (described in clause 5.7.3.7) is set to the Maximum Burst Size of the aggregated TSC streams to be allocated to this QoS flow.  If the AF does not provide a TSC Burst Size for aggregated TSC streams, then the MDBV is set to the default value for the TSC 5QI of the corresponding traffic class.

Table 5.27.2-1: TSC Assistance Information

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The arrival time of the data burst at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

Editor's note:	Which clock the periodicity and burst arrival time refer to is FFS.

END OF CHANGES