3GPP TSG-SA WG2 Meeting #133	S2-1906811

# Reno, USA, 13 - 17 May 2019 	(e-mail revision 4 of S2-1906561)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1306 | rev | 2 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ethernet support clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The limitations of the current Ethernet 5G support are not clear, as discussed in S2-1905040. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that the 5G Ethernet support and the switching behaviour defined for the UPF does not guarantee loop-freenes and the network is not guaranteed to properly react to topology changes. Updates are done for self-consistency of the Ethernet support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limitations of the 5G Ethernet support remain unclear, and the description for Ethernet support is not consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.10.2, 5.8.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 5.6.10.2	Support of Ethernet PDU Session type

For a PDU Session set up with the Ethernet PDU Session type, the SMF and the UPF acting as PDU Session Anchor (PSA) can support specific behaviours related with the fact the PDU Session carries Ethernet frames.

Depending on operator configuration related with the DNN, different configurations for how Ethernet traffic is handled on N6 may apply, for example:

-	Configurations with a 1-1 relationship between a PDU Session and a N6 interface possibly corresponding to a dedicated tunnel established over N6. In this case the UPF acting as PSA transparently forwards Ethernet frames between the PDU Session and its corresponding N6 interface, and it does not need to be aware of MAC addresses used by the UE in order to route down-link traffic.

-	Configurations, where more than one PDU Session to the same DNN (e.g. for more than one UE) corresponds to the same N6 interface. In this case the UPF acting as PSA needs to be aware of MAC addresses used by the UE in the PDU Session in order to map down-link Ethernet frames received over N6 to the appropriate PDU Session. Forwarding behaviour of the UPF acting as PSA is managed by SMF as specified in clause 5.8.2.5.

NOTE 1:	The "MAC addresses used by the UE" correspond to any MAC address used by the UE or any device locally connected to the UE and using the PDU Session to communicate with the DN.

Based on operator configuration, the SMF may request the UPF acting as the PDU Session Anchor to respond to ARP/IPv6 Neighbour Solicitation requests based on local cache information, i.e. the mapping between the UE MAC address to the UE IP address, and the DN where the PDU Session is connected to, or to redirect the ARP traffic from the UPF to the SMF. Responding to ARP/IPv6 ND based on local cache information applies to ARP/IPv6 ND received in both UL and DL directions.

NOTE 2:	Responding to ARP/ND from a local cache assumes the devices behind the UE acquire their IP address via in-band mechanisms that the SMF/UPF can detect.

NOTE 3:	This mechanism is intended to avoid broadcasting or multicasting the ARP/IPv6 ND to every UE.

Ethernet Preamble and Start of Frame delimiter are not sent over 5GS:

-	For UL traffic the UE strips the preamble and frame check sequence (FCS) from the Ethernet frame.

-	For DL traffic the PDU Session Anchor strips the preamble and frame check sequence (FCS) from the Ethernet frame.

Neither a MAC nor an IP address is allocated by the 5GC to the UE for a PDU Session.

The PSA shall store the MAC addresses received from the UE, and associate those with the appropriate PDU Session.

The UPF shall be able remove or reinsert VLAN tags on N6 interface for downlink and uplink frames, respectively, as requested by the SMF. This shall not apply to VLAN tags sent by and received from the UE.

The UE may acquire from the SMF, at PDU Session Establishment, the MTU of the Ethernet frames' payload that the UE shall consider.

NOTE 4:	The UE may operate in bridge mode with regard to a LAN it is connecting to the 5GS, thus different MAC addresses may be used as source address of different frames sent UL over a single PDU Session (and destination MAC address of different frames sent DL over the same PDU Session)

NOTE 5:	Entities on the LAN connected to the 5GS by the UE may have an IP address allocated by the DN but the IP layer is considered as an application layer which is not part of the Ethernet PDU Session.

NOTE 6:	In this Release of the specification, only the UE connected to the 5GS is authenticated, not the devices behind such UE

NOTE 7:	5GS does not support the scenario where a MAC address or if VLAN applies a (MAC address, VLAN) combination is used on more than one PDU Session for the same DNN and S-NSSAI.

NOTE 8:	This release of the specification does not guarantee that the Ethernet network remains loop-free. Deployments need to be verified on an individual basis that loops in the Ethernet network are avoided.

NOTE 9:	This release of the specification does not guarantee that the Ethernet network properly and quickly reacts to topology changes. Deployments need to be verified on an individual basis how they react to topology changes.

Different Frames exchanged on a PDU Session of Ethernet type may be served with different QoS over the 5GS. Thus, the SMF may provide to the UPF Ethernet Packet Filter Set and forwarding rule(s) based on the Ethernet frame structure and UE MAC address(es). The UPF detects and forwards Ethernet frames based on the Ethernet Packet Filter Set and forwarding rule(s) received from the SMF. This is further defined in clauses 5.7 and 5.8.2.

When a PDU Session of Ethernet PDU type is authorized by a DN as described in clause 5.6.6, the DN-AAA server may, as part of authorization data, provide the SMF with a list of allowed MAC addresses and/or a list of allowed VIDs for this PDU Session; the list is limited to a maximum of 16 MAC addresses and/or a maximum of 16 VIDs accordingly. When such list(s) have been provided for a PDU Session, the SMF sets corresponding filtering rules in the UPF(s) acting as PDU Session Anchor for the PDU Session. The UPF discards any UL traffic that does not contain one of these MAC addresses as a source address if the list of allowed MAC addresses is provided. The UPF discards any UL traffic that does not contain one of these VIDs if the list of allowed VIDs is provided.

In this Release of specification, the PDU Session of Ethernet PDU Session type is restricted to SSC mode 1 and SSC mode 2.

For a PDU Session established with the Ethernet PDU Session type, the SMF may, upon PCF request, need to ensure reporting to the PCF of all Ethernet MAC addresses used as UE address in a PDU Session. In this case, as defined in clause 5.8.2.12, the SMF controls the UPF to report the different MAC addresses used as source address of frames sent UL by the UE in the PDU Session.

NOTE 10:	This relates to whether AF control on a per MAC address is allowed on the PDU Session as defined in TS 23.503 [45] clause 6.1.1.2

The PCF may activate or deactivate the reporting of the UE MAC address using the "UE MAC address change" Policy Control Request Trigger as defined in Table 6.1.3.5-1 of TS 23.503 [45].

The SMF may relocate the UPF acting as the PDU Session Anchor for an Ethernet PDU Session as defined in clause 4.3.5.8 of TS 23.502 [3]. The relocation may be triggered by a mobility event such as a handover, or may be triggered independent of UE mobility, e.g. due to load balancing reasons. In order to relocate the PSA UPF, the reporting of the UE MAC addresses needs to be activated by the SMF.

* * * * Next Change * * * *

#### 5.8.2.13	Support for 5G-LAN group communication

In addition to N6-based forwarding of user plane traffic for 5G-LAN group communication, the SMF and UPF may support local switch and Nx-based forwarding:

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G LAN group;

-	Nx-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via Nx. Nx is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

Editor's note:	The way to distinguish the traffic applying N6-based forwarding and traffic applying local switch/Nx-based forwarding in a PDU Session is FFS.

The SMF may configure the UPF(s) to apply different traffic forwarding methods to route traffic between PDU Sessions for a single 5G LAN group. For example, depending on the destination address, some packet flows may be forwarded locally, while other packet flows are forwarded via Nx and other packet flows are forwarded to N6.

The local switch and Nx-based forwarding methods require that a common SMF is controlling the PSA UPFs for the 5G-LAN group.

Editor's note:	The above text may be updated based on eSBA outcome.

To enable local switch in a UPF, the following applies:

-	The SMF provides FARs for 5G LAN group members' N4 Sessions (i.e. N4 Session corresponding to PDU Session) with FAR(s) with Destination Interface set to "5G LAN internal" and Network Instance set to represent the 5G LAN group in order to instruct the UPF to again perform classification of packets using the corresponding 5G LAN group relevant PDRs.

-	The SMF also provides PDRs for 5G LAN group members' N4 Session (i.e. N4 session corresponding to PDU Session) with PDR(s) with Source Interface set to "5G LAN internal" and Network Instance set to represent the 5G LAN group in order to instruct the UPF to detect packets that have been sent to classification via the local switch. This is done by configuring the PDRs in the target 5G LAN group members' N4 Session to includes the destination IP/MAC address(es) of these UEs.

To enable Nx-based forwarding in a UPF, the following applies:

-	The SMF provides FARs for 5G LAN group members' N4 Sessions (i.e. N4 Session corresponding to PDU Session) with FAR(s) with Destination Interface set to "5G LAN Nx" and Network Instance set to represent the 5G LAN group in order to instruct the UPF to again perform classification of packets using the corresponding 5G LAN group relevant PDRs.

-	SMF creates a group-level N4 session to each involved UPF for forwarding packets to a Nx tunnel. The SMF optionally configures the group-level N4 session for processing packets received from a Nx tunnel. Such a N4 session is created per UPF and 5G-LAN group (i.e. a group-level N4 session not corresponding to a PDU Session).

-	The SMF provides relevant N4 rules (e.g. PDRs, QERs, URSs and FARs) for the group-level N4 session applicable for the Nx tunnel(s) in the UPF. The SMF may provides PDRs for the group-level N4 session with Source Interface set to "5G LAN Nx" and Network Instance set to represent the 5G LAN group in order to instruct the UPF to detect packets that are targeting the Nx-based forwarding. In case of multiple Nx tunnels between different PSA UPFs, the SMF may provide N4 rules corresponding to the different Nx tunnels in a single group-level N4 Session.

-	To match the traffic of the UE(s) whose traffic should be forwarded across the Nx tunnel, the PDRs of the group-level N4 Session includes the destination IP/MAC address(es) of these UEs.

-	The SMF may proactively update N4 rules for group level N4-Session to enable correct routing of packets towards UE whose PSA UPF has been reallocated.

-	The SMF may also configure a default PDR in the group-level N4 Session to capture the packets pertaining to 5G LAN group communication with an unknown destination address and a URR to trigger reporting of such events. This enables the SMF to reactively configure relevant PDR and FAR to forward the packets to another UPF via Nx, based on the reports from UPF.

Editor's note:	Whether N4 rules (URRs and FARs) for the group-level N4 session applicable to the traffic applying the local switch/N6-based routing is FFS.

Editor's note:	Handling of multicast traffic with local switch and Nx-based forwarding is FFS.

For Ethernet traffic on 5G-VN, the SMF acts as a central controller which is responsible for setting up the forwarding rules in the UPFs so that it avoids forwarding loops. The SMF becomes aware of the MAC addresses in use within a 5G VN group by the UPF’s reporting of the MAC addresses. The SMF is responsible to react to topology changes in the Ethernet network. Local switching without SMF involvement is not specified for a 5G-VN when different PDU Sessions related with this 5G VN group may be served by different PSA(s) connected over N19.

NOTE:	The mechanisms described above implies signalling on N4 Sessions related with a VN group each time a new MAC address is detected as used (or no more used) within a PDU Session related with this VN group. Hence the usage of the solution defined in this release when raise signalling scalability issues for large VN groups with lots of devices (MAC addresses) served by PDU sessions related with this VN group.

* * * * End of Change * * * *