3GPP TSG-SA2 Meeting #134	S2-1906891

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IPTV Solution Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Juniper Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

The reasons for the changes are discussed in SR-1906892 (IPTV discussions).

* * * First Change * * *

## 9.3	Policy and charging control rule

Policy and charging control rule for 5G-RG PDU Session is described in TS 23.503 [4] clause 6.3 with the clarification and difference shown as below:

-	PCF may take Multicast Access Control list as input to policy decision in case of PDU Session with multicast. PCC rules sent to SMF may indicate allowed IP Multicast traffic as defined in Table 9.3-1.

-	The "Gate status" is not applicable to IGMP/PIM message transmitted over PDU Session with multicast.

Comparing to Table 6.3.1 in TS 23.503 [4], additional PCC rule information for PDU Session for multicast is described in Table 9.3-1.

Table 9.3-1: The additional PCC rule information for PDU Session with multicast

| Information name | Description | Category | PCF permitted to modify for a dynamic PCC rule in the SMF | Differences compared with table 6.3. in TS 23.203 [31] |
| --- | --- | --- | --- | --- |
| Multicast Access Control | This part defines the additional PCC rule information for PDU Session with multicast for multicast access control. |  |  |  |
| IGMP Join control information | Indicates whether the IGMP Join request for an (s,g) or (*,g) is allowed or not | Optional | Yes | Added |
| PIM Join control information | Indicates whether PIM join request for an (s,g) or (*,g) is allowed or not | Optional | Yes | Added |

Editor's note:	PCC Rule for PDU Session with multicast may be reviewed and revised.

* * * Next Change * * *

### 4.6.3	Packet Detection Rule

PDR for 5G-RG PDU Session is described in TS 23.501 [2] clause 5.8.2.11.3 with the clarification shown as below:

About PDU Session used for with multicast,

-	Packets Filter Set support Packet Filters for IGMP messages, including both IGMPv2 specified in RFC 2236 [x] and IGMPv3 specified in RFC 4604 [21] message.

- Packets Filter Set support Packet Filters for PIM messages, as specified in RFC 7761 [z].

Discussion note:	May consider not using PDR/FAR to direct IGMP/[PIM messages to the responsible module. Rather, define a generic procedure for SMF to provision protocols on PDU sessions, which will trigger the protocol module to use local procedures for registering to receive corresponding protocol messages.

### 4.6.4	Forwarding Action Rule

FAR for 5G-RG PDU Session is described in TS 23.501 [2] clause 5.8.2.11.6 with the clarification and difference shown as below:

-	Corresponding to the Policy and charging control rule described in clause 9.3, about PDU Session with multicast, additional "Action" is added:

-	When UPF detects UL IGMP/PIM messages via the PDU Session, sends the messages to a local module responsible for the IGMP/PIM protocol.

Discussion Note: May consider not using PDR/FAR to direct IGMP/[PIM messages to the responsible module. Rather, define a generic procedure for SMF to provision protocols on PDU sessions, which will trigger the protocol module to use local procedures for registering to receive corresponding protocol messages.

* * * Next Change * * *

##### 7.7.1.1.3	Unicast/Multicast Packets transmission procedure

5GS can support Unicast Service from IPTV network directly.

In order to obtain the multicast service from IPTV network, the Multicast Packets transmission procedure should be performed.

Figure 7.7.1.1-3: Multicast Packets transmission procedure

1.	The 5G-RG send the IGMP Join via the user plane.

2.	The UPF detects the IGMP Join message based on PDR received over N4 as described in clause 4.6.3 and sends it to an IGMP or multicast module based on FAR as described in clause 4.6.4. The module determines if the join request is allowed or not according to the multicast access control list it received from SMF

. If it is allowed, the UPF shall add the PDU Session to the requested multicast flow.

In the case of usage IGMPv3 specified in RFC 4604 [21] is configured on the UPF, both Multicast address and Source IP address shall be used for access control by UPF.

* * * Next Change * * *

##### 7.7.1.1.1	Registration and PDU Session Establishment procedure for IPTV

5G-RG perform Registration procedure described in clause 4.2.2.2.2 of TS 23.502 [3] with the following differences:

-	UE is replaced by 5G-RG.

5G-RG perform PDU Session establishment procedure described in clause 4.3.2.2.1 of TS 23.502 [3] applies with the following differences and clarifications:

-	UE is replaced by 5G-RG.

-	In step 1 of clause 4.3.2.2.1, 5G-RG may indicate within the Protocol Configuration Options element that the UE requests to obtain the IPv4 address with DHCPv4.

-	5G-RG shall establish the IP-based PDU Session with a specific DNN for IPTV network.

-	In step 7b and 9 of clause 4.3.2.2.1, PCF provides PCC Rules including QoS Profile(s) related to IPTV Service.

-	In step 10a and 16a of clause 4.3.2.2.1, the parameters for N4 session management may include multicast access control list from SMF to UPF.

NOTE:	The interactions between STB and 5G-RG is going to be specified in TR-124 [5] in BBF and not shown in this clause.

* * * Next Change * * *

## 7.4	SMF and UPF interactions

SMF and UPF interactions for 5G-RG and FN-RG follow the procedures defined in TS 23.502 [3] clause 4.4 with the difference show as below:

About PDU Session with multicast,

-	The parameters N4 session management may include multicast access control list.

- When the access control list changes such that a previous allowed multicast flow is no longer allowed, the UPF shall immediately update its local forwarding state to stop the corresponding flow.

Editor’s Note: For informational purpose, a section should be added in the Annex to describe how relevant multicast modules interact with each other and with PDU sessions.