3GPP TSG-SA2 Meeting #134	S2-1906898

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to NF load data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Impossibility to request the load of a subset of NFs which serve a UE - Missing NFs in the stated list of observable NFs: I-SMF, I-UPF, UPF - Unclear analytics on the NF status (registered, suspended, etc.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Possibility to request the load of a subset of NFs which serves a UE, by providing a list of types of targeted NFs (e.g. AMF, PCF) instead of instance IDs. - Add to list of possible NFs types: I-SMF, I-UPF and UPF - Add the types of instance in the output - Add analytics per value of NF status (registered, suspended, undiscoverable) - Various text rephrasing for clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obligation for an application to know the instance IDs before requesting the NWDAF or for understanding of the outputs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1, 6.5.2, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## 6.5	NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics Id set to "NF load information";

-	The Target of Analytics Reporting: the SUPI;

-	Analytics Filter Information: list of Instance IDs or list of instance types;

-	An Observation period indicates the time when the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If the NF Instance IDs are provided, the NWDAF shall provide the analytics for each designated NF instance. In such case the Target of Analytics Reporting should be ignored.

If the NF Instance IDs are not provided, the NWDAF shall use the specific SUPI to determine which instances are presently serving this specific UE. The list of optional instance types may be provided in order to restrict the results.

The list of possible instances types is limited to AMF, N3IWF, SMF or I-SMF, and PCF, UPF or I-UPF, UDM.

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1.

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF load | NRF | The load of a specific NF instance in the NF profile as defined per TS.29.510 [18]. |
| NF status | NRF | The status of a specific NF instance (registered, suspended, undiscoverable) as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for a specific NF instance (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7. |

Editor's note:	It is FFS whether the number of registered instances of an NF is needed input data for NWDAF to perform predictions on NF instance load and resource usage.

NOTE:	The OAM information can be used as a complement to NRF information on resources, or as a way to correlate NRF information or as an alternative if NRF information on load is not available.

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition with Table 6.5.2-1.

Table 6.5.2-2: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

### 6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Load statistics information is defined in Table 6.5.3-1.

Table 6.5.3-1: NF load statistics

| Information | Description |  |
| --- | --- | --- |
| Resource status (1..n) | List of observed load information for each NF instance |  |
| > Instance type | Type of the instance as defined in clause 6.5.1 |
| > Instance ID | Identification of the NF instance |  |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |  |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |  |
| > NF load | The average load of the NF instance over the observation period |  |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |  |

-	Load predictions information is defined in Table 6.5.3-2.

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| Resource status (1..n) | List of predicted load information for each NF instance |
| > Instance type | Type of the instance as defined in clause 6.5.1 |
| > Instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the observation period |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |
| > Confidence | Confidence of this prediction |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

* * * End of changes * * * *