3GPP TSG-SA WG2 Meeting #134 	S2-1906928

Sapporo, Japan, 24 – 28 June, 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1462 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling RRC connection reestablishment for Control Plane for NB-IoT connected to 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIOT |  |  |  |  |  | Date: |  |  | 2019-06-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has requested feedback from SA2 in incoming LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC (R2-1908264). Two aspects are requested from RAN2: - RAN2 would like to ask SA3 and SA2 on the feasibility of introducing RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC and if found feasible to introduce RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC in their specifications. - RAN2 would like to inform SA3 and SA2 that, due to the limited size of the message, if the same security procedure and token (ul-NAS-MAC and ul-NAS-Count) as in EPS is used, there is not sufficient space to carry the 5G S-TMSI and a truncated 5G S TMSI (e.g. 40 bits) may need to be defined. This contribution provides the techincal specification normative text in line with the proposed way forward in Discussion paper S2-1908264. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce truncated 5G-S-TMSI defintion for use in RRC reestablihment procedure in NB-IoT. - Introduce reference to ng-eNB CP Relocation Indication Procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for RRC connection reestablishment procedure for NB-IoT, impacting enhanced mobility in NB-IoT when connected to 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.X (new), 5.31.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

## 5.9.X	Truncated 5G-S-TMSI

The Truncated 5G-S-TMSI is a 40 bit UE identifier constructed from the 5G-S-TMSI. It is used in  RRC Connection Re-Establishment for the control plane for NB-IoT as described in TS 38.300 [].

<Truncated 5G-S-TMSI> := <Truncated AMF set ID><Truncated AMF Pointer><Truncated 5G-TMSI>

<Truncated AMF set ID> = :n LSBs of AMF Set ID, where n is no greater than 10 bits.

<Truncated AMF Pointer> := m LSBs of AMF Pointer, where m is no greater than 6 bits.

<Truncated 5G-TMSI> := (40-n-m) LSBs of 5G-TMSI.

The values n and m are configurable based on network deployment. The value n+m shall be larger or equal to 8 bits.

NOTE: 	Depending on network deployment it is up to operator configuration to ensure that Truncated AMF Set ID and Truncated AMF Pointer identify the AMF uniquely, and that Truncated 5G-TMSI identifies the UE uniquely within the serving AMF.

The NG-RAN is configured with the values n and m, and it is configured with how to recreate AMF Set ID from Truncated AMF Set ID, AMF Pointer from Truncated AMF Pointer, and 5G-TMSI from Truncated 5G-TMSI.  The configuration of these parameters are specific to each PLMN.

The NG-RAN configures the UE with n and m during RRC connection resonfiguration as described in TS 38.331 [28]. The configuration applies only to the registered PLMN.

*** NEXT CHANGES***

### 5.31.X	ng-eNB CP Relocation Indication Procedure

For intra-NB-IoT mobility for UE and AMF using Control Plane CIoT 5GS Optimisation the ng-eNB CP Relocation Indication procedures may be used. The purpose of the eNB CP Relocation Indication procedure is to request the AMF to authenticate the UE's re-establishment request as described in TS 33.501 [], and initiate the establishment of the UE's N2 connection and, if necessary, the N3 connection after the UE has initiated a RRC Re-establishment procedure in a new NG-RAN.

More details are defined in TS 38.300 [27].

*** END CHANGES ***