3GPP TSG SA2 Meeting #134	S2-1907056

# Sapporo, Japain, June 24th - 28th , 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0014 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session related policy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 18-06-2019 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDU session policy control information needs to be extended to include the possibility to provide the Authorized Session-TMBR for all GBR and non GBR flows transferred over wireline non-3GPP access network |  |  |  |  |  |  |  |  |
| Summary of change: |  | # The PDU session Policy control indudes the Authorized Session-TMBR and subsequent values, to provide a similar solution as for the Authorized Session AMBR, to avoid large signalling when the calendar time changes this velue for a large amount of users. # The SMF signals the Session-TMBR to UPF and the access network for enforcement, referencing to clause 4. # The proposal is that: # - a single access PDU session over non 3GPP wireline access, the UPF enforces the Session-TMBR. # - a multiaccess PDU session over non 3GPP wireline access and 3GPP access, the UPF enforces Session AMBR, |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of the Session-TMBR policy control |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x, 8.x.1, 8.x.2,8.x.3, 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 9.4	PDU Session related policy information

This clause specifies the delta related to PDU session related policy information defined in TS 23.503 [4] clause 6.4 for 5G-RG and FN-RG.

Table 9.4-1: Wireline access specific PDU Session related policy information

| Attribute | Description | PCF permitted to modify for dynamically provided information | Scope |
| --- | --- | --- | --- |
| Authorized Session-TMBR (NOTE 1) (NOTE 2) | Defines the Total Maximum Bit Rate for the GBR and Non-GBR QoS Flows of the PDU Session transmitted over non-3GPP wireline access. | Yes | PDU Session |
| Time Condition (NOTE 3) | Defines the time at which the corresponding Subsequent Authorized Session-TMBR | No (NOTE 4) | PDU Session |
| Subsequent Authorized Session-TMBR (NOTE 3) (NOTE 1) | Defines the Total Maximum Bit Rate for the GBR and Non-GBR QoS Flows of the PDU Session transmitted over non-3GPP wireline access when the Time Condition is reached | No (NOTE 4) | PDU Session |
| NOTE 1: The Authorized Session-TMBR and the Subsequent Authorized Session-TMBR is provided for Access Type “non-3GPP access” and Access Technology Type “wireline”. NOTE 2: There may be an unconditional value for the Authorized Session-TMBR at the SMF. The initial value is received as Subscribed Session-TMBR, if the UE has a wireline subscription and the PCF can overwrite it with these parameters. NOTE 3: The Time Condition and Subsequent Authorized Session-TMBR are used together. The PCF may provide up to four instances of them. When multiple instances are provided, the values of the associated Time Condition have to be different. NOTE 4: The PCF may replace all instances that have been provided previously with a new instruction. A previously provided Time Condition and Subsequent Authorized Session-TMBR cannot be individually modified. |  |  |  |

The Authorized Session-TMBR defines the UL/DL Total Maximum Bit Rate for the GBR and Non-GBR QoS Flows of the PDU Session transmitted over non-3GPP wireline access, which is enforced in the UPF as defined in clause 4.5.1.1. The PCF may provide the Authorized Session-TMBR in every interaction with the SMF. When the SMF receives it from the PDU Session policy, it is provided to the UPF over N4 interface and to the Wireline Access for the enforcement.

The Time Condition and Subsequent Authorized Session-TMBR are used together and up to four instances with different values of the Time Condition parameter may be provided by the PCF. Time Condition indicates that the associated Subsequent Authorized Session-TMBR is only applied when the time defined by this attribute is met. When the SMF receives a Time Condition and Subsequent Authorized Session-TMBR pair, it stores it locally. The SMF shall discard any previously received Subsequent Authorized Session-TMBR instances as well as whenever the PCF provides a new instruction for one or more Subsequent Authorized Session-TMBR. When the time defined by the Time Condition parameter is reached, the SMF shall instruct the UPF to apply the Subsequent Authorized Session-TMBR.

When the PDU session is associated with a non-3GPP wireline as single access, the SMF provides the Authorized Session-TMBR for enforcement to UPF.

When the PDU session is associated both non-3GPP wireline and 3GPP access, the SMF provides the Authorized Session-AMBR for enforcement to UPF.

The PCF may subscribe to Access Type and RAT/Technology Type change to the SMF, depending on operator policies, the PCF may e.g. update the Authorized Session-TMBR value to ensure that matches the Session-AMBR and sum of GBR flows in the PDU session or update the Authorized Session-AMBR for a multiaccess PDU session to take into account GBR flows.

Next change

## 8.x	Npcf_SMPolicyControl service

### 8.x.1	Npcf_SMPolicyControl_Create service operation

The input data listed in clause 5.2.5.2.2 in TS 23.502 [3] applies when an SM Policy Association is created for a 5G-RG or a FN-RG, in addition the following input and output parameters applies:

Inputs, Required: None

Inputs, Optional: Session-TMBR

Outputs, Required: None

Outputs, Optional: Policy information for the PDU Session as defined in clause 9.4.

### 8.x.2	Npcf_SMPolicyControl_UpdateNotify service operation

The input data listed in clause 5.2.5.2.3 in TS 23.502 [3] applies when an SM Policy Association is created for a 5G-RG or a FN-RG, in addition the following input and output parameters applies:

Inputs, Required: None

Inputs, Optional: Policy information for the PDU Session as defined in clause 9.4.

Outputs, Required: None

Outputs, Optional: None.

### 8.x.3	Npcf_SMPolicyControl_Update service operation

The input data listed in clause 5.2.5.2.2 in TS 23.502 [3] applies when an SM Policy Association is created for a 5G-RG or a FN-RG, in addition the following input and output parameters applies:

Inputs, Required: None

Inputs, Optional: Session-TMBR.

Outputs, Required: None

Outputs, Optional: Policy information for the PDU Session as defined in clause 9.4.

End of changes