3GPP TSG-SA WG2 Meeting #134	S2-1907118

# Sapporo, Japan, 24 June – 28 June 2019 	(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0017 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document addresses the specification of Network Functions defined in TS 23.503 clause 6.2. The PCF and SMF behavior have been extended in order to support the 5G-RG and FN-RG in TS 23.316. These extensions need to be reflected in the specification of PCF and SMF currently defined in TS 23.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 9.2.1 General New clause 9.2.2 defining the delta of PCF for supporting WWC scenario New clause 9.2.3 defining the delta of SMF for supporting WWC scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncomplete specification and support of 5G-RG and FN-RG connected via W-5GAN.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 9.2	Network Functions and entities

### 9.2.1 General

This clause specifies the delta related to Network Function and entities defined in TS 23.503 [4] clause 6.2 for 5G-RG and FN-RG.

### 9.2.2 PCF

The specification defined in TS 23.503 clause 6.2.1 applies for 5G-RG and FN-RG with the following modifications:

-	PCF provides the Access and mobility related policy control as described in clause 9.5.1

-	PCF provides the UE access selection and PDU Session selection related policy control as defined in 9.5.2.

-	On receiving the Session-TMBR provided by the SMF at the establishment or modification of the SM Policy Association for a PDU session under the conditions defined in TS 23.503 clause 5.6.6, the PCF provides the Authorized Session AMBR as part of the PDU session policy control information defined in clause 9.4

The specification defined in TS 23.503 clause 6.2.1 applies for 5G-RG and FN-RG with the following modifications:

-	The 5G-RG and FN-RG replaces the UE.

NOTE : In case of FN-RG the W-AGF acts on behalf of FN-RG, however from PCF point of view the W-5AGF is not distinguishable from the FN-RG.

The specification defined in TS 23.503 clause 6.2.3 applies for 5G-RG and FN-RG with the following modifications:

-	The 5G-RG and FN-RG replaces the UE

-	Roaming is not applicable in this release.

## 9.2.3 Session Management Function (SMF)

The specification defined in TS 23.503 clause 6.2.2 applies for 5G-RG and FN-RG with the following modifications:

-	The SMF may support functionality for traffic steering, switching and splitting within a MA PDU Session, as described in TS 23.501 [2] with the modification in clause 4.12 in case of MA PDU session used in the Hybrid Access scenario

**** End of changes ****