3GPP TSG-SA2 Meeting #134	S2-1907327

# Sapporo, Japan, 24th June 2019 - 28th June 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0027 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications of the pre-check behaviours of the NF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph2 |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Two ENs are left during the procedure for NWDAF assisted misused or hijacked UEs identification: Editor´s note: Whether new event ID(s) or Event Filters needs to be defined for reporting UE behaviour deviating from its expected UE behaviour is FFS. Editor´s note: Whether new event ID(s) or Event Filters needs to be defined for reporting UE behaviour deviating from its expected UE behaviour is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add an indicator in the Namf_EventExposure_Subscribe or the Nsmf_EventExposure_Subscribe to indicate the AMF or SMF should pre-check whether the UE's behaviour matches its expected UE behavioural information. If no, then AMF or SMF should sends event reports to the NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The two ENs are not resolved in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.7.5.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

6.7.5.4	Procedure

Figure 6.7.5.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a.	A consumer NF subscribes to NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID set to "Abnormal behaviour", Analytics Filter(s) = Internal-Group-Identifier or SUPI).

A consumer NF may subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b.	AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Analytics Filter(s) = External-group identifier or External UE ID).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsSubscription_Subscribe (Analytics ID, Analytics Filter(s) = External-group-identifier or External UE ID).

An AF may also subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE, where the subscription message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier.

2.	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Event Filter=(Internal-Group-Identifier or SUPI, pre-check indicator)).

The NWDAF sends subscription requests to the related AMF to collect UE behavioural information if it has not subscribed such data.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

Based on the pre-check indicator, the AMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the AMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

3.	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Event Filter=(Internal-Group-Identifier or SUPI, pre-check indicator)).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

Based on the pre-check indicator, the SMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the SMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator's policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription) (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, Internal-Group-Identifier or SUPI, Exception level)).

If the NWDAF determines to send a notification to the subscribed 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify services. Based on the notification, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.7.5.3-1.

5b.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, External UE ID)).

If the NWDAF determines to send a notification to the subscribed AF, the NWDAF needs to include external UE ID of the identified UE into the notification message.

NOTE 3:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

**** End of Change ****