3GPP TSG-SA WG2 Meeting #134	S2-1907434

# Sapporo, Japan,  June 24 - June 28, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0034 | rev | 0 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on Potential QoS Change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Remove FFS on description of Potential QoS Change and update parameters descriptions for subscriptions to reflect requirements of Notification of Potential QoS Changes feature of TS23.287. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove FFS in Clause 6.9.1; Clarify text about location information parameters included in the subscription/request to NWDAF, so that these parameters can be directly used by NWDAF without requiring further NWDAF capabilities to translate/convert such data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requriements of Notification of Potential QoS Changes feature of TS23.287 are not fully supported in Rel. 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 6.9.2, 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 6.9	Potential QoS Change analytics

### 6.9.1	General

The consumer may request the NWDAF analytics information regarding potential QoS change in a geographic area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters:

-	Analytics ID = "Potential QoS change".

-	Analytics Filter Information:

-	QoS requirements:

-	Standardized 5QI, and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR)

-	For non-standardized QoS Characteristics, the QoS Characteristics attributes PDB and PER and their values (optional)

-	Location information: could be a path of interest, or a form of Geographical Area(s), e.g. geographical coordinates or a polygon describing an area. The location information could reflect a list of waypoints.

NOTE 1: In this release, the consumer of the “Potential QoS Change” analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) which is understandable by NWDAF.

-	Time information (optional): could be relative time interval that indicates the time period for which the analytics about the potential QoS change is requested. If not provided, the NWDAF uses a default value set by the operator based e.g. on local policy or SLA.

-	Threshold(s): The threshold(s) indicate level(s) which, if crossed, trigger the notification that the potential QoS change (improvement or worsening) can happen. The level(s) relate to the QoS KPIs, i.e the average RAN UE Throughput, the average QoS flow Retainability, etc., for the relevant 5QI(s) defined in TS 28.55 [10]:

-	for a 5QI of GBR resource type, the Threshold(s) refer to the average QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Threshold(s) refer to the average RAN UE Throughput KPI;

-	S-NSSAI (optional).

The NWDAF collects the corresponding statistics information on the QoS KPIs of interests from the OAM.

The NWDAF verifies whether the triggering conditions for the notification about potential change in QoS are met and if so, generates for the consumer one or more notifications about potential change in QoS.

The potential QoS change detected by the NWDAF is based on data from the OAM, e.g., the average QoS flow Retainability, the average RAN UE Throughput , etc., for the relevant 5QI(s) as defined in TS 28.554 [10], and for parameters in the request from the consumer.

The NWDAF detects the need for notification about "potential QoS change" based on comparing the expected value for the KPI of the target 5QI against the threshold(s) provided by the consumer in any cell in the requested geographic area for the requested time information. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

The NWDAF provides notification on potential QoS change to the consumer. The analytics feedback contains the information on the location in the geographic area and the time when a potential QoS change may occur, as well as information on the expected QoS KPI with the respect to the provided threshold(s), i.e. whether the expected QoS KPI is above or below each of the Threshold(s).

* * * * Start of 2nd Change * * * *

### 6.9.2	Input data

Table 6.9.2-1: Data collection for Potential QoS Change analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | How often the UE abnormally loses a QoS Flow during the time the QoS Flow is used, per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

* * * * Start of 3rd Change * * * *

### 6.9.3	Output analytics

The analytics feedback contains the following information:

-	Information on the location in the geographic area and the time when a potential QoS change may occur.

- 	Information on the expected QoS KPI with the respect to the provided threshold(s), i.e. whether the expected QoS KPI is above or below each of the Threshold(s).

* * * * End of Changes * * * *