3GPP TSG-SA WG2 Meeting #134	S2-1907624

# Sapporo, Japan, June 24 – 28, 2019										 (revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1622 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | De-activating user-plane resources over one access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CN-initiated deactivation of UP connection procedure is used to deactivate UP connection for an established single access PDU session of a UE in CM-CONNECTED state. The procedure should be extended in the case of one access of the MA PDU Session to support ATSSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CN-initiated deactivation of UP connection procedure is used when the network wants to de-activate the user-plane resources over one access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network cannot de-activate the user-plane resources over one access of the MA PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.7	Adding / Re-activating / De-activating User-Plane Resources

If the UE has established a MA PDU Session but the user-plane resources over one access of the MA PDU Session have not been established, then:

-	If the UE wants to add user-plane resources over this access, the UE shall initiate the UE Requested PDU Session Establishment procedure over this access, as specified in clause 4.3.2.2. The PDU Session Establishment Request message shall contain the "MA PDU Request" indication and the PDU Session ID of the established MA PDU Session.

-	The PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules for the MA PDU session.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established but are currently inactive (e.g. because the UE is CM-IDLE over this access), then:

-	If the UE wants to re-activate the user-plane resources over this access, then the UE shall initiate the Registration or UE Triggered Service Request procedure over this access, as specified in clause 4.22.9.1 and clause 4.22.8.2, or clause 4.12.4.1.

-	If the network wants to re-activate the user-plane resources over this access and the access is available, then the network shall initiate the Network Triggered Service Request procedure over this access, as specified in clause 4.2.3.3.

Editor's note:	It is FFS if and how the network can request the re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established and are currently active (e.g. the UE is CM-CONNECTED over this access), then:

-	If the network wants to de-activate the user-plane resources over this access, then the network shall initiate the CN-initiated deactivation of UP connection procedure over this access, as specified in clause 4.3.7.

* * * * End of Changes * * * *