3GPP TSG-SA WG2 Meeting #134	S2-1907882

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-1907565)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1607 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of access addition – Option 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE is registered in both accesses in the same PLMN, during the MA PDU Session Establishment procedure, the SMF tries to establish a user plane resources in both accesses. In this case, if the MA PDU Session is established in home routed mode or I-SMF is inserted, the V-SMF or I-SMF may fails to setup user plane resources due to e.g. UE is CM-IDLE in non-3GPP access or UE is in non-allowed area in 3GPP access. In this case, the UE will send PDU Session Establishment message to add second access. Then (H-)SMF receives PDU Session Establishment message even though assoicated SM contexts are already exist. In order to solve this issue, it is proposed that the SMF provides an information on whether the MA PDU Session is established over a single access or both accesses in PDU Session Establishment Accept message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the SMF receives PDU Session Establishment request message while the SMF already has associated SM contexts, the SMF re-use existing SM contexts and performs user plane activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF may receive PDU Session Establishment request message when the assoicated SM contexts are already created. It is not clear how the SMF should hand such case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.7	Adding / Re-activating User-Plane Resources

If the UE has established a MA PDU Session but the user-plane resources over one access of the MA PDU Session have not been established, then:

-	If the UE wants to add user-plane resources over this access, the UE shall initiate the UE Requested PDU Session Establishment procedure over this access, as specified in clause 4.3.2.2. The PDU Session Establishment Request message shall contain the "MA PDU Request" indication and the PDU Session ID of the established MA PDU Session.

-	The PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules for the MA PDU session.

-	If the SMF receives the PDU Session Establishment request message over an access and the SMF already has SM Contexts for the access, the SMF shall not release existing SM Contexts and shall re-activate user plane resources over the access while providing the PDU Session Establishment Accept message to the UE.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established but are currently inactive (e.g. because the UE is CM-IDLE over this access), then:

-	If the UE wants to re-activate the user-plane resources over this access, then the UE shall initiate the Registration or UE Triggered Service Request procedure over this access, as specified in clause 4.22.9.1 and clause 4.22.8.2, or clause 4.12.4.1.

-	If the network wants to re-activate the user-plane resources over this access and the access is available, then the network shall initiate the Network Triggered Service Request procedure over this access, as specified in clause 4.2.3.3.

Editor's note:	It is FFS if and how the network can request the re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session.

* * * * End of Changes * * * *