SA WG2 Meeting #134	S2-1908029

# 24 - 28 June 2019, Sapporo, Japan	 										(revision of S2-1907588)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0022 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 3GPP access specific aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.2 3GPP access specific aspects is still empty. Most of the features and procedures defined in TS 23.273 are common for both 3GPP access and non 3GPP access. Features related with the access type majorly include the positioning method and access type selection, both of which were already defined. The related description can be restructured to ensure no overlapped definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Positioning methods description is restructured to refer to 5.2 and 5.3 for each access specific positioning methods. Add an the description to 5.2 to address the 3GPP access specific aspects to be considered and refer to relavent TS or clauses for details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear about the 3GPP access specific aspects for location service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2b, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.2b	Positioning methods

The LCS feature utilises one or more positioning methods in order to determine the location of user equipment (UE). Determining the position of a UE involves two main steps:

-	Radio signal measurements or non-RAT measurements; and

-	Position estimate computation based on the measurements.

The positioning methods for 3GPP access  are described in clause 5.2.

The positioning methods for non-3GPP access are described in clause 5.3.1.

NEXT CHANGE

## 5.2	3GPP access specific aspects

When 3GPP access type is selected, the positioning methods for 3GPP access defined in TS 38.305 [9] apply.

Access Type Selection for LCS Service is defined in clause 5.3.2.

END OF CHANGES