3GPP TSG-SA WG2 Meeting #134 	S2-1908048

Sapporo, Japan, June 24-28, 2019 	(revision of S2-1908030)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1539 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SM context transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections to the SM Context Transfer procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to modify the procedure description according to the changes below and remove the current editors notes Rev 1 Merge with S2-1906972 Removed first EN “It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.” Updated the sequence diagram accordingly. The SMF becomes in control of the context transfer instead of the AMF. This could be beneficial in some cases, e.g., when deadlines for context transfer are to be met and the timing information should be managed solely by the Old SMF. Removed second EN: “It is FFS if step 6 and 9 can be optimised or changed to a modification of the N4 session”. Introduced “”N4 session release” with flag notifying the UPF about the expected re-establishment of the N4 session for the same PDU session. Based on this, the UPF may temporarily keep some information related to the N4 session to optimize packet handling until the N4 session is re-established by the new SMF. Rev 1 Merge with S2-1907128, S2-1907216, S2-1907315, S2-1907523, S2-1907663 Updated 4.26.5.3 to clarify the target SMF can be selected dynamically in consideration of the actual network status with the use of the target SMF set ID. Updated 4.26.5.1 to clarify that the SMF context transfer procedure can be used between two SMFs within the same SMF set. Change Nsmf_PDUSession_StatusNotify to Nsmf_PDUSession_SMContextStatusNotify in clause 4.26.5.2. In clause 4.26.5.3, In step 2, change Nsmf_PDUSession_StatusNotify to Nsmf_PDUSession_SMContextStatusNotify. For the second editor’s note, it is proposed to delete it and change N4 session establish to N4 session modification and change SM policy association creation to update. In the original figure, step 6 and 13 are removed due to the revision in the above bullet. Update the service description for Nsmf_PDUSession_SMContextStatusNotify Added Old SMF ID, PDU Session ID, SUPI, SM Context ID tp CreateSMContext request so as to be consistent with the information sent in step 1. Clarified the UE state as CM-IDLE and CM-CONNECTED with RRC inactive state Added notification from New SMF to Old SMF to notify the finishing of the context transfer. Clarified that not “everything” is removed from the UPF but rather only unused information Clarification in Nsmf_PDUSession_ContextRequest service operation to mention SM Context Transfer How it is handled if concurrent request from peer NFs comes to the old SMF while the session is transferring to the new SMF; How the new SMF interaction with PCF; How the old registration and subscription in UDM is handled; How the N4 session is handled; Updated CreateSMContext service operation Rev 2 Replaced SMContextStatusNotify with notification from UDM, which was missing and can also serve the same purpose. Adjusted text in 4.XX.4, procedure text and figure to reflect this change Text in 10.1 and 10.2 was not matching the figure. Aligned text with the depicted procedure Moved “The parameters from step 8 and, if applies, step 9 are used.” (step 10.2) to step 10.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less optimal solution for SMF context transfer Imprecise specification Notification not designed for subscription by AMF used for SM Context Transfer System and old SMF are not aware of the state of the transfer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.XX.4, 4.26.5, 5.2.3.2.2, 5.2.8.2.5, 5.2.8.2.8, 5.2.8.2.10, 5.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.xx.4	Context Operations During Context Transfer

Figure 4.XX.4-1: Race condition handling for NF/NF Service Context Transfer procedure

- During context transfer procedures (e.g. SM context transfer), if Source NF/NF Service receives from a NF/NF Service Consumer a service request operating on a context being transferred, e.g. PCF or UDM or UPF (A.1), it rejects the service request with an indication that context transfer is ongoing (A.2).

Per the indication received in A.2, NF/NF Service Consumer may have a locally configured guard timer. At expiry of the guard timer, NF/NF Service Consumer may reattempt the same service request with

B. Target NR/NF Service (B.2) if it receives the new context location from B.1 (e.g. query to UDM or SMContextStatusNotification in case of SM Context Transfer).

C. Source NF/NF Service (C.2): Source NF/NF Service can redirect the request of NF/NF Service Consumer to the new location of the transferred context (C.3) if it received it in C.1 (e.g. UDM notification with new SMF ID in case of SM Context Transfer).

*** NEXT CHANGES ***

### 4.26.5	SMF Service Context Transfer procedures

#### 4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

#### 4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_SMContextStatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that, if the UE IP address is received from Old SMF, the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the concerned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM to Old SMF including SUPI, PDU session ID and New SMF ID or SMF set ID. The SMF selection by using SMF set ID not applicable when the IP range is managed by SMF.

2.	[Conditional – depending on current subscription] Old SMF subscribes to events when UE status becomes CM-IDLE or CM-CONNECTED with RRC inactive state (Namf_EventExposure_Subscribe).

3.	[Conditional – depending on the event] The AMF detects the monitored event occurs and sends the event report by means of Namf_EventExposure_Notify message, to Old SMF.

4.	From Old SMF to AMF Nsmf_PDUSession_SMContextStatusNotify (SMF transfer indication, Old SMF ID, New SMF ID or SMF set ID from Step 1, PDU Session ID, SUPI, SM Context ID).

5.	AMF or SCP if delegated discovery is used uses New SMF ID or SMF set ID to select New SMF and sends Nsmf_PDUSession_CreateSMContext request (PDU Session ID, Old SMF ID, SM Context ID in Old SMF, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 4 is used.

6.	From New SMF to Old SMF SMF Nsmf_PDUSession_ContextRequest request (SM Context type, SM Context ID, SMF transfer indication). If New SMF is not capable to transfer this SM Context (e.g. it is not responsible for the IP range), steps 9 to 12 are skipped.

7.	Old SMF releases the N4 session with the UPF by sending a flag notifying the UPF about the expected re-establishment of the N4 session for the same PDU session. Based on this, the UPF may, based on implementation, delay the release of the N4 session up to step 10.2 to allow for uninterrupted packet handling until the N4 session is re-established by New SMF.

8.	From Old SMF to New SMF Nsmf_PDUSession_ContextRequest response containing SM Context. The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet as well as the UPF to be selected by New SMF. Old SMF starts a timer to monitor the SMF context transferring process.

9.	[Conditional] If dynamic PCC is used for the PDU Session, New SMF updates the policy association towards PCF.

10.1. UPF receives a N4 session establishment request for the same PDU session from step 7. The parameters from step 8 and, if applies, step 9 are used.

10.2. UPF performs a full re-establishment of the session (IMS signaling, online charging etc.), establishing a new N4 session. All information related to the N4 session of Old SMF that is not used by the N4 session of New SMF  is removed from UPF if not already done.

11. New SMF registers to UDM. The information stored at the UDM includes SUPI, SMF identity and the associated DNN and PDU Session ID.

12.	New SMF subscribes to subscription changes for the UE.

13. From New SMF to AMF: Nsmf_PDUSession_CreateSMContext response. If this response indicates a redirect (e.g. another SMF in the set), the procedure moves to step 5 with the indicated endpoint as target.

14.	UDM notifies Old SMF that it is deregistered for the PDU Session by sending Nudm_UECM_DeregistrationNotification, optionally including New SMF ID

15. [Conditional] If 14 was not received and the timer from step 8 expires, Old SMF re-establishes the N4 session. The UPF may for the purpose use the information stored in step 7. In this case, the procedure ends here.

16.	[Conditional] If 14 was received, Old SMF removes its policy association with PCF. Any changes to the QoS rules need to be sent to the UE when it becomes active.

17. Old SMF releases any internal resources corresponding to the indicated PDU session. Subscribers to SMContextStatusNotify for the transferred SM context are notified of the context transfer and optionally of the new location of the transferred SM context.

18. [Conditional] Old SMF may redirect incoming requests (18a) to New SMF (18b) based on the information received in 14.

*** NEXT CHANGES ***

##### 5.2.8.2.8	Nsmf_PDUSession_SMContextStatusNotify service operation

Service operation name: Nsmf_PDUSession_SMContextStatusNotify.

Description: This service operation is used by the SMF to notify its consumers about the status of an SM context related to a PDU Session (e.g. PDU Session release due to local reasons within the SMF, PDU Session handover to a different system or access type). The SMF may use this service operation to update the SMF derived CN assisted RAN parameters tuning in the AMF.

Input, Required: Status information.

Input, Optional: Cause, SMF derived CN assisted RAN parameters tuning, New SMF ID for SM Context Transfer (see 4.26.5.3).

Output, Required: Result Indication.

Output, Optional: None.

*** NEXT CHANGES ***

##### 5.2.3.2.2	Nudm_UECM_DeregistrationNotification service operation

Service operation name: Nudm_UECM_DeregistrationNotification.

Description: UDM notifies the NF consumer which has previously registered (using Nudm_UECM_Registration operation) has been deregistered in the UDM. As a result, the consumer is no longer registered in UDM as a serving NF for that UE.

NOTE:	This notification corresponds to an implicit subscription.

Inputs, Required: SUPI, Access Type, PDU Session ID, serving NF deregistration reason.

Inputs, Optional:NF ID in case of SM Context Transfer.

Outputs, Required: None.

Outputs, Optional: None.

See step 14d of clause 4.2.2.2.2, 4.26.5.3 for an example usage of this service operation. The serving NF deregistration reason tells the reason for sending the deregistration notification to the consumer NF.

The reason for AMF deregistration can be one of the following:

-	UE Initial Registration.

-	UE Registration area change.

-	Subscription Withdrawn.

-	5GS to EPS Mobility.

The reason for SMF deregistration can be one of the following:

-	SMF Context Transfer.

*** NEXT CHANGES ***

##### 5.2.8.2.5	Nsmf_PDUSession_CreateSMContext service operation

Service operation name: Nsmf_PDUSession_CreateSMContext.

Description: It creates an AMF-SMF association to support a PDU Session.

Input, Required: SUPI or PEI, DNN, AMF ID (AMF Instance ID).

Input, Optional: PEI, S-NSSAI(s), PDU Session Id, N1 SM container, UE location information, UE Time Zone, AN type, H-SMF identifier/address, list of alternative H-SMF(s) if available, old PDU Session ID (if the AMF also received an old PDU Session ID from the UE as specified in clause 4.3.5.2), Subscription For PDU Session Status Notification, indication that the SUPI has not been authenticated, PCF ID, DNN Selection Mode, UE PDN Connection Context, GPSI, UE presence in LADN service area, GUAMI, backup AMF(s) (if NF Type is AMF), Trace Requirements, Control Plane CIoT 5GS Optimisation indication. Backup AMF(s) sent only once by the AMF to the SMF in its first interaction with the SMF, UE's Routing Indicator or UDM Group ID for the UE, EPS Bearer Status. Target ID (for EPS to 5GS handover), additional following three for SM context transfer: SMF transfer indication, Old SMF ID, SM context ID in old SMF (see 4.26.5.3).

Output, Required: Result Indication, and if successful SM Context ID.

Output, Optional: Cause, PDU Session ID, N2 SM information, N1 SM container, S-NSSAI(s).

When the PDU Session is for Emergency services for a UE without USIM, the AMF provides the PEI and not the SUPI as identifier of the UE. When the PDU Session is for Emergency services of an unauthenticated UE with an USIM, the AMF shall provide both the SUPI and the PEI and shall provide an indication that the SUPI has not been authenticated.

See clause 4.3.2.2.1, clause 4.3.2.2.2, clause 4.11.1.2.2 and clause 4.11.1.3.3 for details on the usage of this service operation.

*** NEXT CHANGES ***

##### 5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO, SM Context transfer indication), or during mobility procedure with I-SMF changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, PDU Session ID (include PDU Session ID when available).

Output, Required: SM Context Container.

Output, Optional:Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requrested, e.g. PDN Connection Context ,5G SM Context or both. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context that may be transferred between I-SMF(s) or between V-SMF(s)

| Field | Description |
| --- | --- |
| SUPI | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |
| Trace reference | Identifies a record or a collection of records for a particular trace. |
| Trace type | Indicates the type of trace |
| OMC identity | Identifies the OMC that shall receive the trace record(s). |
| S-NSSAI | The requested S-NSSAI for the serving PLMN to the PDU Session. |
| HPLMN S-NSSAI | Requested S-NSSAI for the HPLMN to the Home-Routed PDU Session |
| Network Slice Instance id | The network Slice Instance information for the PDU Session |
| DNN | The associated DNN for the PDU Session. |
| AMF Information | The associated AMF instance identifier and GUAMI. |
| Access Type | The current access type for this PDU Session. |
| RAT Type | RAT Type used by the UE. |
| PDU Session ID | The identifier of the PDU Session. |
| H-SMF Information | The associated H-SMF identifier and H-SMF address for the HR PDU Session.( applies only for a V-SMF) |
| For each QoS Flow in the PDU Session: |  |
| 5G QoS Identifier (5QI) | Identifier for the authorized QoS parameters for the service data flow. |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |
| GFBR | Guaranteed Flow Bit Rate (GFBR) - UL and DL |
| MFBR | Maximum Flow Bit Rate (MFBR) - UL and DL |
| Priority Level | Indicates a priority in scheduling resources among QoS Flows. |
| Averaging Window | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |
| Maximum Data Burst Volume | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |
| Reflective QoS Control | Indicates to apply reflective QoS for the SDF in the TFT. |
| QoS Notification Control (QNC) | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |
| Maximum Packet Loss Rate | Maximum Packet Loss Rate - UL and DL. |
| Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |  |
| EPS Bearer Id | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN |
| Mapped EPS Bearer QoS | ARP, GBR, MBR, QCI. |
| PGW-U tunnel Information | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |
| TFT | Traffic Flow Template |

Editor's note:	This table is known to be incomplete and to require updates. Some items listed in the table may be removed. This table requires also updates to cover all kind of SM contexts not limited to I-SMF and V-SMF.

*** END CHANGES ***