3GPP TSG-SA WG2 Meeting #134	S2-1908061

# Sapporo, Japan, 24 – 28 June 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1607 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update P-CSCF Discovery using NRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.228 supports several methods for P-CSCF discovery (and selection), but the text of clause 5.16.3.11 implies that only one of these methods is used for 5GC – however the other alternatives are still possible. Current functionality for S-GW selection of P-CSCF described in 23.228 specifies that the criteria used for p_CSCF discovery is implementation specific – since the APN/DNN could be used to select different P-CSCF pools based on the APN acecss method (e.g. one pool for Internet APN and another for IMS APN) it is determined that using DNN in the 5GC procedure should be supported – however the current positioning of the text for DNN implies this is a primary factor and not an edge case – thus the order of the list needs to be adjusted before the EN can be removed. According to current functionality for EPC to discover (and select) the P-CSCF during IP-CAN establishment only IP addresse(s) of P-CSCF(s) are communicated to the UE – to avoid UE confusion for 5GC this should also be the case, however currect text is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that clause 5.16.3.11 is applicable to to the determination of the P-CSCF discovery method performed during IP-CAN establishment. Removes Editor’s Note on use of DNN in P-CSCF Discovery. Moves DNN to bottom of list of factors that may be used. Clarifies that if the NRF returns FQDNs to the SMF, that only IP addresses are sent to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. Undetermined if DNN can be used in selection of P-CSCF. UE’s mistakenly sent FQDN of P-CSCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | TDOC S2-1907502 – Original CR TDOC S2-1908061 – Revised to remove all added text to first paragraph (not shown in revision marks to avoid changes over changes) – a simple “may be used” is inserted to clarify that other methods of P-CSCF selection are not prevented. |  |  |  |  |  |  |  |  |

< **** First Change ****>

#### 5.16.3.11	P-CSCF discovery and selection

P-CSCF selection functionality may be used by the SMF to select the P-CSCF for an IMS PDU Session of the UE.

The SMF can utilize the Network Repository Function to discover the P-CSCF instance(s). The NRF provides the IP address or the FQDN of P-CSCF instance(s) to the SMF. The P-CSCF selection function in the SMF selects the P-CSCF instance(s) based on the available P-CSCF instances obtained from NRF or based on the configured P-CSCF information in the SMF. If the SMF receives FQDN(s) from the NRF or is configured with FQDN(s) the SMF shall resolve these to IP addresses for sending to the UE in the PDU session response.

The following factors may be considered during the P-CSCF discovery and selection:

-	S-NSSAI of the PDU Session.

-	UE location information.

-	Local operator policies.

-	Availability of candidate P-CSCFs.

-	UE IP address.

-	Access Type.

-	Selected Data Network Name (DNN).

< **** End of Changes ****>