3GPP TSG-SA WG2 Meeting #134 	S2-1908314

Sapporo, Japan, 24 – 28 June, 2019	(revision of S2-1906923)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1460 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for UP optimization with EDT procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIOT |  |  |  |  |  | Date: |  |  | 2019-06-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As proposed in S2-1906921, Resume procedure with EDT should be able to carry a NAS PDU with data payload for PDU sessions that are using CP CIoT 5GS optimisation (i.e. no user plane resources for the PDU session were active when the UE entered CM-IDLE with Suspend state). This needs to be clarified in TS 23.501. Also, it is benefitial to avoid duplication of text to reference TS 23.502 Resume procedures, since many of the details of the procedure with EDT and without EDT are captured in TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Resume procedure with EDT can carry NAS PDU with data payload - Reference TS 23.502 Resume procedure for details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear that Resume procedure with EDT can carry NAS PDU with data payload, preventing clear coexistence of CP optmisation and UP optimisation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 4.8.2.X is added by TS.502 CR#1462 |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.31.18	User Plane CIoT 5GS Optimisation

User Plane CIoT 5GS Optimisation enables transfer of user plane data from CM-IDLE without the need for using the Service Request procedure to establish Access Stratum (AS) context in NG-RAN and UE.

If the following preconditions are met:

-	UE and AMF negotiated support User Plane CIoT 5GS Optimisation (see clause 5.31.2) over NAS,

-	the UE has indicated support of User Plane CIoT 5GS Optimisation in the UE radio capabilities as defined in TS 36.331 [51],

-	AMF has indicated User Plane CIoT 5GS Optimisation support to NG-RAN,

-	the UE has established at least one PDU session with active UP connection, i.e. AS context is established in NG-RAN and the UE,

then the RRC connection can be suspended by means of the Connection Suspend Procedure (see clause 4.8.1.2 of TS 23.502 [3]).

Based on a trigger from the NAS layer when a UE is in CM-IDLE with Suspend, the UE should attempt the Connection Resume in CM-IDLE with Suspend procedure (clause 4.8.2.3 of TS 23.502 [3]). If the Connection Resume in CM-IDLE with Suspend procedure fails, the UE initiates the pending NAS procedure. To maintain support for User Plane CIoT 5GS Optimisation for UE mobility across different NG-RAN nodes, the AS Context should be transferred between the NG-RAN nodes, see TS 38.300 [27] and TS 38.423 [99].

By using the Connection Suspend Procedure:

-	the UE at transition into CM-IDLE stores the AS information;

-	NG-RAN stores the AS information, the NGAP UE association and the PDU session context for that UE;

-	AMF stores the NGAP UE association and other information necessary to later resume the UE, interacts with the SMF(s) to deactivate the user plane resources for the UE's PDU Sessions and enters CM-IDLE.

NG-RAN may decide based on implementation to delete the stored UE context and NGAP association. In that case, the RAN shall initiate the AN Release procedure as described in clause 4.2.6 of TS 23.502 [3]. NG-RAN does not initiate any RRC procedure to notify the UE of the UE context release.

By using the Connection Resume in CM-IDLE with Suspend procedure:

-	the UE resumes the connection from CM-IDLE with the network using the AS information stored during the Connection Suspend procedure;

-	the NG-RAN node notifies the AMF that the connection with the UE has been resumed and the AMF enters CM-CONNECTED and interacts with the SMF to activate the user plane resources for the UE's PDU Sessions.

The UE may include early uplink data or early NAS PDU with data payload in the Connection Resume.

Connection Resume procedure without Early Data Transfer is described in TS 23.502 [3] clause 4.8.2.3.

Connection Resume procedure with Early Data Transfer is described in TS 23.502 [3] clause 4.8.2.X.

If the AMF establishes an NGAP UE association with a new NG-RAN node different from the stored NGAP UE association, e.g. the UE initiates service request or registration procedure from a different NG-RAN node, the AMF initiates UE N2 release command towards the old NG-RAN node.

NG-RAN maintains the N3 tunnel endpoint information while a UE is in CM-IDLE with Suspend. UPF is instructed to remove DL N3 Tunnel Info of AN during Connection Suspend procedure, while UPF keeps UL N3 Tunnel Info (i.e. UPF accepts and forwards UL data).

*** END CHANGES ***