3GPP TSG SA WG2 Meeting #134	S2-1907029

# Sapporo, Japan, June 24 - 28, 2019															(Rev of)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1487 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UEs not supporting 5GC NAS connected to PGW-C+SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#133, 23.502 CR1328 (S2-1906779) addressing the 4G only UEs connected to PGW-C+SMF was agreed. #1 The new text is placed under clause for EPC-5GC interworking, however a UE not supporting 5GS NAS will never support 5GC interworking. #2 PDU Session is not required in UPF. #3 For a UE not supporting 5GC NAS connected to PGW-C+SMF which interacts with UDM and PCF, IP preservation is not supported when UE moves to GERAN/UTRAN. #4 A UE’s 5GC NAS capability may be disabled and re-enabled, which mean that both UE and the PGW-C+SMF may assign PDU Session ID. #5 Currently PDU Session ID has value 1-15 as defined in TS 24.007, considering that both UE and PGW-C may allocate PDU Session, to ensure its uniqueness, it’s proposed that the PGW-C+SMF use an extended value range. To avoid UE impact, it’s proosed that the extended value range is only applicable for CN. DP S2-1907028 may be referenced for more discussison. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the scenario for UEs not supporting 5GC NAS connected to the PGW-C+SMF in a separate clause, proposed structure as follows: 4.11.5a Impact to EPC procedure to support UEs not using 5GC NAS 4.11.5a.1 General 4.11.5a.2 Impact to EPS procedure 4.11.5a.3 Impact to EPC/ePDG procedures 4.11.5a.4 PGW-C+SMF assigning PDU Session ID Remove “interaction with UPF” Add a note that IP address is not preserved if PGW-C+SMF is selected for a UE not supporting 5GC NAS and interaction with UDM/PCF is required moves to GERAN/UTRAN. PGW-C+SMF use extended range of PDU Session ID and this range is only used in CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/misleading specification; UEs that do not have 5GC NAS capability may not be able to establish PDN connection. UEs with N1 mode capabiltiy disabled and then re-enabled may not be able to establish PDN connection due to duplicated PDU Session ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.1, 4.11.5a (new), 4.11.5a.1, 4.11.5a.2, 4.11.5a.3, 4.11.5a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of changes ***************

### 4.11.0a	Impacts to EPS Procedures

#### 4.11.0a.1	General

This clause captures changes to procedures in TS 23.401 [13] that are common to interworking based on N26 and interworking without N26.

*************** Next changes ***************

#### 4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. In case the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values.

*************** Next changes ***************

### 4.11.5a Impact to EPC procedures for UEs not supporting 5GC NAS

#### 4.11.5a.1 General

This clause captures the changes to procedures in TS 23.401 [13] and procedures in TS 23.402 [26] if PGW-C+SMF is selected and interaction with UDM and PCF is required for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN or ePDG.

#### 4.11.5a.2	Impact to EPS procedure

The procedure as specified in TS 23.401 [13] applies with the following modification:

- 	The PGW-C+SMF creates a PDU Session ID for the PDN connection as specified in clause 4.11.5a.4 for the following procedures:

Step 13 of clause 5.3.2.1 (E-UTRAN Initial Attach);

Step 3 of clause 5.10.2

-	PCRF interaction in TS 23.401 [13] is replaced with PCF interaction as specified in clauses 4.11.0a.2.

IP addresss is not preserved when UE not supporting 5GC NAS moves from E-UTRAN to GERAN/UTRAN.

#### 4.11.5a.3	Impact to EPC/ePDG procedures

The procedure as specified in TS 23.402 [26] applies with the following modification:

The PGW-C+SMF creates a PDU Session ID for the PDN connection as specified in clause 4.11.5a.4 in the following procedures:

Clause 7.2.4 Initial Attach with GTP on S2b:

Step B.1

Clause 7.6.3 UE-initiated Connectivity to Additional PDN with GTP on S2b:

#### 4.11.5a.4	PGW-C+SMF assigning PDU Session ID

At PDN connection establishment from a UE not supporting 5GC NAS via MME or via ePDG, the PGW-C+SMF shall ensure that the PDU Session ID assigned to the PDN connection is not duplicated by using an extended range that is only used in the NFs.

NOTE: For a UE, the PDU Session ID can be assigned by both the UE and the PGW-C+SMF.

*************** End of changes ***************