3GPP TSG-SA2 Meeting #134	S2-1907313

# Sapporo, Japan, June 24 – 28, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1538 | rev | 0 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of 4G-only UEs with SMF/PGW-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 06-17-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2 #133, it was agreed to allow that SMF/PGW-C can provide services to 4G-only UEs if the subscription does not have a restriction with 5GC access. To enable this feature, it should be possible for the SMF/PGW-C to determine whether the UE is 5G-capable or not without any ambiguity (see S2-1907311 for more information). In the current specification, it was described that MME selects a PGW for the UE among only PGW-C+SMF and PGW-C (with CUPS), without considering the deployment without CUPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 4.11.0a.5 to clarify that SMF/PGW-C determines the 5G capability of the UE by using the indication from the UE in PCO (Protocol Configuration Option). Updated 4.11.0a.4 to clarify that the MME can also select a standalone PGW (without CUPS) if there is no need to support 5G-4G interworking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operation of UE and SMF/PGW-C is not clear to support 4G-only UEs with 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.5, 4.11.0a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. In case the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values.

If PGW-C+SMF is selected and interaction with UDM, PCF and UPF is required for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN or ePDG, the PGW-C+SMF creates unique PDU Session ID for each PDN connection of the UE. The PGW-C+SMF shall not provide any 5GS related parameters to the UE. The PGW-C+SMF determines whether the UE supports 5GC NAS or not by using the indication provided by the UE via PCO.

NOTE:	The unique PDU Session ID can be created from the EPS Bearer IDs assigned by the MME or the ePDG for the PDN Connections associated with the UE.

SECOND CHANGE

4.11.0a.4	PGW Selection

When the UE requests to establish a PDN connection to an APN, the MME may use the UE's support for 5GC NAS indication included in the UE Network Capability and/or UE's subscription from HSS that includes UE's mobility restriction paramters related to 5GS and/or indication of support for interworking with 5GS for this APN to determine if PGW-C+SMF or a standalone PGW/PGW-C should be selected.

NOTE:	If restriction for Core Network Type indicates that the UE can access to 5GC, it implies that the UE has 5G subscription data.