3GPP TSG-WG SA2 Meeting #135 	S2-1908873

# Split, Croatia, October 14 – 18, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1714 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MA PDU Upgrade in modification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSlicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To keep alignment with TS 23.502, a single PDU Session can be converted to an MA PDU Session during PDU Session Modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe that a single PDU Session can be converted to an MA PDU Session during PDU Session Modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The single PDU session cannot be converted to the MA PDU session in the PDU Session Modification procedure, resulting in the ATSSS functionality cannot be fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

5.32.2	Multi Access PDU Sessions

A Multi-Access PDU (MA PDU) Session is managed by using the session management functionality specified in clause 5.6, with the following additions and modifications:

-	When the UE wants to request a new MA PDU Session:

-	If the UE is registered to the same PLMN over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over any of the two accesses containing a "MA PDU Request" indication. The AMF informs the SMF that the UE is registered over both accesses and this triggers the establishment of user-plane resources on both accesses and two N3/N9 tunnels between PSA and the RAN/AN.

-	If the UE is registered to different PLMNs over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over one access containing a "MA PDU Request" indication. After this PDU Session is established with one N3/N9 tunnel between the PSA and (R)AN established, the UE shall send another PDU Session Establishment Request over the other access containing also a "MA PDU Request" indication and the same PDU Session ID. Two N3/N9 tunnels and User-plane resources on both accesses are established.

-	If the UE is registered over one access only, then the UE shall send a PDU Session Establishment Request over this access containing a "MA PDU Request" indication. One N3/N9 tunnel between the PSA and (R)AN and User-plane resources on this access only are established. After the UE is registered over the second access, the UE shall establish user-plane resources on the second access.

-	In the PDU Session Establishment Request that is sent to request a new MA PDU Session, the UE shall provide also its ATSSS capabilities, which indicate whether it is capable of supporting the ATSSS-LL functionality, or the MPTCP functionality, or both (these functionalities are defined in clause 5.32.6).

-	If the UE indicates it is capable of supporting the MPTCP functionality and the network accepts to activate the MPTCP functionality, then the network provides MPTCP proxy information to UE, and allocates to UE one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and two additional IP addresses/prefixes, called "link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.

-	If the UE indicates it is capable of supporting the ATSSS-LL functionality and the network accepts to activate the ATSSS-LL functionality, then the network may provide to UE Measurement Assistance Information (see details in clause 5.32.5).

-	If the UE requests an S-NSSAI, this S-NSSAI should be allowed on both accesses. Otherwise, the MA PDU Session shall not be established.

-	The PCC rules provided by PCF include ATSSS control information (see 23.503 [45]). They are used by SMF to derive ATSSS rules for the UE and N4 rules for the UPF.

-	The UE receives ATSSS rules from SMF, which indicate how the uplink traffic should be routed across 3GPP access and non-3GPP access. Similarly, the UPF receives N4 rules from SMF, which indicate how the downlink traffic should be routed across 3GPP access and non-3GPP access.

-	After the MA PDU Session establishment:

-	At any given time, the MA PDU session may have user-plane resources on both 3GPP and non-3GPP accesses, or on one access only, or may have no user-plane resources on any access.

-	The AMF, SMF, PCF and UPF maintain their MA PDU Session contexts, even when the UE deregisters from one access (but remains registered on the other access).

-	When the UE deregisters from one access (but remains registered on the other access), the AMF informs the SMF that an access type becomes unavailable for the MA PDU Session. Subsequently, the SMF notifies the UPF that the access type has become unavailable and the N3/N9 tunnel for the access type are released.

-	If the UE wants to add user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall send a PDU Session Establishment Request over this access containing PDU Session ID of the MA PDU Session and a "MA PDU Request" indication. If there is no N3/N9 for this access, the N3/N9 tunnel for this access is established.

-	If the UE wants to re-activate user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall initiate the UE Triggered Service Request procedure over this access.

-	If the network wants to re-activate the user-plane resources over 3GPP access or non-3GPP access of the MA PDU Session, the network shall initiate the Network Triggered Service Request procedure, as specified in TS 23.502 [3], clause 4.22.7.

A MA PDU Session may be established either:

a)	when it is explicitly requested by an ATSSS-capable UE; or

b)	when an ATSSS-capable UE requests or modifies a single-access PDU Session but the network decides to establish a MA PDU Session instead. This is an optional scenario specified in TS 23.502 [3], clause 4.22.3 and clause 4.22.6.3, which may occur when the UE requests or modifies a single-access PDU Session but no policy (e.g. no URSP rule) and no local restrictions in the UE mandate a single access for the PDU Session.

An ATSSS-capable UE may decide to request a MA PDU Session based on the provisioned URSP rules. In particular, the UE should request a MA PDU Session when the UE applies a URSP rule, which triggers the UE to establish a new PDU Session and the Access Type Preference component of the URSP rule indicates "Multi-Access" (see TS 23.503 [45]).

* * * * End of Change * * * *