3GPP TSG-WG SA2 Meeting #135 	S2-1908928

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0038 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Line ID uniqueness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The LS from BBF in S2-1908655 (BBF Liase-310) has provided information regarding the usage of Line ID and its uniqueness. BBF concluded that : 1) in order to ensure the uniqueness of Line ID sourced from multiple access provider while preserving the line ID information within their system, BBF reccomands to add a field identifying the access provider (5 digit is suggested) to the Line ID econded as SUPI. 2) operator today may not use all information encoded in Line ID, so it is reccomended that the components extracted from Line ID information for SUPI contruction be selectable. The usage of PLMN ID will guarantee the uniqueness, althought BBF considers applicable for roaming which is not considered for wireline in Rel-16. BBF reccomends that Line ID is encoded with a viariable length opaque object of up to 135 bytes. However a greater length is welcomed for future usage. The document proposed to introduce the defintion of a new Global Line identifier and of Global Cable identifier which are composed by the Line ID plus operator identifier and HFC indetifier and operator identifier respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed revision is to introduce a new Global Line Idenfier and a Global Cable Identifier. The Global Line Idenfier is composed by the Line ID and the operator identifier of the operator administrating the W-AGF, while the Global Cable Idenfier is composed by the operator identifier of the operator administrating the HFC_Identifier and the HFC_Identifier value. The detailed of the definition of format of these idenfiers would be specified in TS 23.003 and in relevant BBF and CableLabs specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct specification of Line ID and Cable HFC_Identifier support in W-5GAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 4.7.2, 4.7.3, 4.7.x (new), 4.7.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.7.1	General

As described in TS 23.501 [2], each subscriber in the 5G System shall be allocated one 5G Subscription Permanent Identifier (SUPI) for use within the 3GPP system. As described in TS 23.501 [2], each FN-RG or 5G-RG accessing the 5G System shall be assigned a Permanent Equipment Identifier (PEI).

The clauses below describe specific aspects for supporting 5G-RG and FN-RG.

Editor's note:	Further analysis on how to ensure that HFC_Identifier is unique is FFS. For example, scenarios for wholesale as well as scenarios where an operator manages multiple overlapping spaces of Line ID or HFC_Identifier values should be addressed and are FFS.

### 4.7.2	SUPI and SUCI for 5G-BRG support

The SUPI for an 5G-BRG shall contain an IMSI, as described in TS 23.501 [2], clause 5.9.2.

The SUCI provided by the 5G-BRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

### 4.7.3	SUPI and SUCI for FN-BRG support

The SUPI for an FN-BRG subscription shall contain either an IMSI or a Global Line  idenfier defined in clause 4.7.x, based on operator configuration.The encoding of SUPI from Line ID shall be configurable by operator.

The SUCI provided by the W-AGF to the 5GC for FN-BRG support always contains a Global Line Idedntifier defined in clause 4.7.x. If the SUPI associated with the subscription for FN-BRG contains an IMSI, the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-BRG contains a Global Line Identifier, there is no need for a mapping in UDM.

As described in TS 23.003 [14], the SUCI also contains an identifier of the Home network.

Editor's note:	Additional details regarding SUCI, including whether or not a Line ID in SUCI is concealed, is to be determined by SA WG3. The text above about SUCI for wireline access may need to be updated based on SA WG3 outcome.

### 4.7.4	SUPI and SUCI for 5G-CRG and FN-CRG support

The SUPI for a 5G-CRG/FN-CRG subscription shall contain either an IMSI, as described in TS 23.501 [2], clause 5.9.2, or an Global Cable Identifier defined in clause 4.7.y..

Editor's note:	The use of non-IMSI based SUPI and associated credentials and authentication method needs to be specified by SA WG3.

The SUCI provided by the 5G-CRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

The SUCI provided to the network for FN-CRG support always contains a HFC_Identifier together with an operator identifier of the operator administrating the HFC_Identifier value. If the SUPI associated with the subscription for FN-CRG contains an IMSI, the HFC_Identifier included in the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-CRG contains a HFC_Identifier, there is no need for a mapping in UDM.

As described in TS 23.003 [14], the SUCI also contains an identifier of the Home network for both IMSI and network-specific identifier based SUPI. When SUCI is derived from network-specific identifier (taking the form of a NAI), the username shall contain the HFC_Identifier and the realm part shall identify the home network of the operator administrating the HFC_Identifier value.

Editor's note:	Additional details regarding SUCI, including whether or not a HFC_Identifier in SUCI is concealed, is to be determined by SA WG3. The text above about SUCI for wireline access may need to be updated based on SA WG3 outcome.

### 4.7.5	Line ID

The format of the Line ID is generic, allowing different operators and access networks to use different encoding of the content. The Line ID can have the following formats:

-	DHCPv4 option 82 exchange (TR-101 issue 2 [6] Annex B, original specification 2006);

-	PPPoE Circuit and Remote ID AVP insertion (TR-101 issue 2 [6] Annex A/8.3, original specification 2006);

-	DHCPv6 use of option 18 via LDRA functionality in the access node (BBF TR-177 Issue 1 Corrigendum 1 [12], original specification 2010);

-	Use of the Line ID Option (LIO) in RS messaging (BBF TR-177 Issue 1 Corrigendum 1 [12] , original specification 2010). Also documented in RFC 6788 [13], which differs slightly from the BBF encodings.

### 4.7.x	Global Line Identifier

For the scope of usage with 5GC in SUPI or SUCI, a Global Line Identifier is specified in order to define a globally unique identifier which is composed the operator identifier of the operator administrating the W-AGF and the Line ID value. The identifier of operator administrating the W-AGF and the Line ID should be based on PLMN ID, if applicable, or on opertor identifier defined by BBF.

The Global Line Idenfier is a variable length identifier encoded as defined in TS 23.003 and in BBF WT-456.

### 4.7.y	Global Cable Identifier

For the scope of usage with 5GC in SUPI or SUCI, a Global Cable Identifier is specified in order to define a globally unique identifier which is composed the operator identifier of the operator administrating the HFC_Identifier and the HFC_Identifier value. The identifier of operator administrating the W-AGF and the HFC_Identifier should be based on PLMN ID, if applicable, or on opertor identifier defined by BBF.

The Global Cable Idenfier is a variable length identifier encoded as defined in TS 23.003 and by Cablelabs in [x].

# * * * * End of changes * * * *