SA WG2 Meeting #S2-135	S2-1909009

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0056 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User Data Load Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 clarified in response LS in S2-19xxxxxx that “performance measurements defined in TS 28.552 represent resource utilisation but do not indicate the event of congestion or congestion levels”. Hence the procedure, based on the incorrect understanding that RAN PM provide congestion information, currently specifying a ‘congestion analytics’ needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure is updated and the term ‘congestion’ replaced by ‘load’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text leading to misunderstanding of the properties of the procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1, 6.8.2, 6.8.3, 6.8.4.1, 6.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

## 6.8	User Data Load Analytics

### 6.8.1	General

The NWDAF can provide user data load related analytics, in the form of statistics or predictions or both, to another NF. User Data Load related analytics can relate to load experienced while transferring user data over the control plane or user plane. A request for user data load analytics relates to a specific area or to a specific user. If requestor provides a UE ID, the NWDAF determines the area where the UE is located at the time of request. The NWDAF then collects measurements per cell and uses the measurements to determine user data load analytics.

When a one-time report of user data load analytics is desired, the requesting NF invokes the Nnwdaf_AnalyticsInfo service. When continuous reporting of user data load analytics is desired, the requesting NF invokes the Nnwdaf_AnalyticsSubscription service.

The request for user data load related analytics indicates the location area information where load related analytics is desired or indicates a UE Identity that can be used by the NWDAF to determine the location area information where load related analytics is desired. The request indicates whether user data load statistics, predictions, or both are desired. The type of analytics is set to user data load analytics for transfer over the user plane, for transfer over the control plane, or for both. When the requestor subscribes to user data load related analytics, it may indicate a threshold and the NWDAF will provide analytics to the requestor when the load level crosses the threshold.

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

Table 6.8.2-1: Service Data from the NF related to User Data Load Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE Location | AMF | UE location information that maybe provided to the NWDAF when the NWDAF is used derive the Area of Interest. |
| AMF ID | UDM | The AMF ID to be used for obtaining the UE's location. |
| Performance Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine load levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). |

Editor's note:	The exact performance measurements (or KPIs) that are collected from the OAM need to be coordinated with SA5.

### 6.8.3	Output analytics

The NWDAF outputs the user data load analytics for transfer over the user plane, for transfer over the control plane, or for both. The output may consist of statistics, predictions, or both. The detailed information provided by the NWDAF is defined in Table 6.8.3-1.

Table 6.8.3-1: User Data Load Analytics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAI or Cell IDs |
| Analytics |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication (NSI) | Load Level |

NOTE:	When the output analytics consist of predictions and the NWDAF uses a UE Identity to determine the location area information, the predictions are associated with the UE's location at the time of the request.

### 6.8.4	Procedures

#### 6.8.4.1	Procedure for one-time or continuous reporting of analytics for user data load in a geographic area

The procedure as depicted in Figure 6.8.4.1-1 is used by an NF to retrieve load analytics for a specific geographic area. The procedure can be used to request a one-time or continuous reporting of load analytics.

Figure 6.8.4.1-1: Procedure for one-time or continuous reporting of analytics for load in a geographic area

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, indicating request for analytics for load in a specific location. The NF can request statistics or predictions or both. The type of analytics is set to user data load analytics for transfer over user plane, control plane, or both, Analytics Filter is set to a location (e.g. ECGI, TA).

2-3. If the request is authorized, and in order to provide the requested analytics, the NWDAF may request the load status for the requested location from OAM services following the data collection from OAM procedure as captured in 6.2.3.2. If the NWDAF already has information about the load status for the requested location, these steps are omitted.

Editor's note:	The exact SA5 service used is FFS.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics for load to the NF.

For continuous reporting:

6.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF, indicating request for analytics for load in a specific location (e.g. ECGI, TA), possibly with thresholds. The NF can request statistics or predictions or both. The type of analytics is set to user data load analytics for transfer over user plane, control plane, or both.

7-8. The NWDAF subscribes to OAM services following the data collection from OAM procedure as captured in 6.2.3.2 to get the user data load status for the requested location, possibly providing load level thresholds.

Editor's note:	The exact SA5 service used is FFS.

9.	The NWDAF derives requested analytics.

10.	The NWDAF provides the analytics for load to the NF.

11.	A change of user data load status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

12.	The NWDAF derives new analytics.

13.	The NWDAF provides a notification for analytics for the user data load to the NF.

#### 6.8.4.2	Procedure for one-time or continuous reporting of analytics for user data load for a specific UE

The procedure as depicted in Figure 6.8.4.2-1 is used by an NF to retrieve user data load analytics for a specific UE. The procedure can be used to request a one-time or continuous reporting of user data load analytics.

Figure 6.8.4.2-1: Procedure for one-time or continuous reporting of analytics for load for a specific UE

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, requesting for analytics for user data load for a specific UE id. The NF can request statistics or predictions or both. The type of analytics is set to user data load analytics for transfer over user plane, control plane, or both, the Target of Analytics Reporting is set to UE id.

2-5. The NWDAF may already know the UE location. If not, the NWDAF checks the UE location by first retrieving the AMF serving the UE (steps 2-3) and then by interrogating the AMF about the UE location.

6-7. The NWDAF requests the user data load status for the UE location from OAM services (as captured in 6.2.3.2). These steps are omitted if the NWDAF already has the information.

Editor's note:	The exact SA5 service used is FFS.

8.	The NWDAF derives requested analytics.

9.	The NWDAF provides the analytics for load to the NF.

For continuous reporting:

10.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF. The NF can request for statistics or for predictions or for both. The type of analytics is set to user data load analytics for transfer over user plane, control plane, or both.

11.	The NWDAF determines the UE location, either via internal information or by applying the same steps as steps 2 to 5. The NWDAF then determines an area of interest.

12-13. The NWDAF subscribes to OAM services (as captured in 6.2.3.2) to get the user data load status for the UE location, possibly providing load level thresholds.

Editor's note:	The exact SA5 service used is FFS.

14.	The NWDAF derives requested analytics.

15.	The NWDAF provides the analytics for user data load status information to the NF.

16-17. The NWDAF subscribes to UE mobility event notification in order to be informed when the UE moves out of the area of interest (in order to define a new area of interest and request new information to OAM if the UE moves to a different area).

18.	A change of user data load status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

19.	The NWDAF derives new analytics.

20.	The NWDAF provides a notification for analytics for the user data load status information to the NF.

*** END CHANGES ***