SA WG2 Meeting #135 	S2-1909016

# October 14-18th, 2019 ; Split, HR		                               			 (revision of S2-1909016)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0039 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UDM/UDR subscription data support for a mapping from Line ID to the the SUPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 23.316 already specifies (for FN RG) The SUCI provided by the W-AGF to the 5GC for FN-BRG support always contains a Line Id together with an operator identifier of the operator administrating the Line ID value. If the SUPI associated with the subscription for FN-BRG contains an IMSI, the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-BRG contains a Line Id, there is no need for a mapping in UDM. This means that the UDM shall contain a mapping from Line ID to the the SUPI |  |  |  |  |  |  |  |  |
| Summary of change: |  | UDM/UDR subscription data support a mapping from Line ID to the the SUPI: Other Identifier Of The Subscription (5GC Line ID). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Following Feature cannot be supported: the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 8.1.1.1	General

In addition to the Subscription data types used in the Nudm_SubscriberDataManagement Service, as defined in Table 5.2.3.3.1-1 of TS 23.502 [3], the additional data types defined in Table 8.1.1.1-1 below are applicable for RGs connected to 5GC via W-5GAN.

Table 8.1.1.1-1: Wireline access specific UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| Access and Mobility Subscription data (data needed for UE Registration and Mobility Management) | RG Level Wireline Access Characteristics | The RG level Wireline Access Characteristics parameter provides QoS information for the W-AGF, as defined in clause 4.5.1.2. This parameter is handled by the UDM as a transparent container. |
|  | Other Identifier Of The Subscription (5GC Line ID) | If the SUPI associated with the subscription for a FN-BRG contains an IMSI, the SUCI which contains a Line ID acts as pseudonym of the SUPI and the UDM performs a mapping from the Other Identifier Of The Subscription (5GC Line ID) to the SUPI of the subscription (that contains an IMSI). |
| Session Management Subscription data (data needed for PDU Session Establishment) | Subscribed-Session-TMBR | The maximum aggregated uplink and downlink MBRs to be shared across all GBR and Non-GBR QoS Flows in each PDU Session, which are established for the DNN, S-NSSAI. Applicable only for the wireline access network for a 5G-RG or FN-RG, as defined in clause 4.4.x.1. |

In case of Wireline access, the Forbidden area information within Table 5.2.3.3.1-1 of TS 23.502 [3] may correspond to a (set of) allowed 5GC Line ID.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES