q3GPP TSG SA WG2 Meeting #135	S2-1909026

# Split, Croatia, October 14 - 18, 2019													Rev of S2-19xxxxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1224 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discovery of Transport in P-CSCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2019-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently in case of EPS fallback, there is a very high probability that SIP signaling will be lost during RAT redirection. Lost SIP signaling can be recovered at the SIP level, if UDP is used as a transport mechanism for SIP signaling, or at the transport level, through use of TCP as the transport mechanism for SIP signaling. SIP timers used in SIP signaling recovery at the SIP level, are generally longer than the transport recovery timers. In addition, these SIP recovery timers are not uniformly set across all implemenations since they are guidelines for a range, as opposed to explicit recovery times. Hence to avoid session setup delays due to SIP signaling recovery, TCP must be the transport used. With the current P-CSCF discovery procedure in 5GC, only IP-addresses of the P-CSCFs are returned to the UE. There is no indication of the port and preferred transport protocol type (UDP/TCP) together with the P-CSCF address. In 5GS, TCP is preferred and desired during EPS FB, and as such, including the supported transport or prefered transport enables the UE to choose the right P-CSCF. This information is currently not provided to the UE and is needed to enable the UE to select a P-CSCF with TCP support for EPS fallback. If UDP is selected, then SIP recovery faults will be handled at the SIP signaling level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the support for the P-CSCF to register the the supported transport type (TCP/UDP or both), the corresponding port number with priority indication for each transport type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Longer IMS session times that will cast a negative consumer experience of 5G as a new technology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.1.1, Y.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

## Y.1.1	Procedures for P-CSCF discovery

All the procedures described in clause 5.1.1 apply with the following additions:

-	For the option where P-CSCF discovery is part of the IP-CAN connectivity establishment the following applies:

-	P-CSCF discovery shall take place during the PDU session establishment procedure see TS 23.502 [94], clause 4.3.2.

In addition to the procedures in clause 5.1.1, the SMF may determine the P-CSCF using service-based discovery methods (using the NRF), as described in TS 23.501 [93] clause 5.16.3.

The UE shall select a P-CSCF from the most prioritized one to least the prioritized.

NOTE:	The UE can receive a prioritized list of P-CSCF IP addresss and port.  For example, if the TCP is perfered by the network, P-CSCFs with TCP discovered has higher priority in NRF during the discovery. In this case, the UE selects a P-CSCF to use in the following order: P-CSCF supporting TCP as a priority transport scheme for SIP signalling, followed by a P-CSCF supporting TCP as a lower priority transport scheme for SIP signalling, followed by a P-CSCF supporting UDP regardless of the priority.

*************** Next Change ***************

# Y.12	P-CSCF Registration in NRF

In order to support service based SMF discovery of the P-CSCF using the NRF (as described in TS 23.501 [93] clause 5.16.3) the P-CSCF’s in a network will need to register with an applicable NRF.  When a network uses other P-CSCF discovery methods (as described in clause 5.1.1) the P-CSCF does not need to register with the NRF. Local configuration of the P-CSCF is used to determine if the P-CSCF shall perform registration with the NRF.

When the P-CSCF is configured to support SMF discovery of the P-CSCF, P-CSCF shall register in NRF their capabilities using the Nnrf_NFManagement_NFRegister Request message. The NF profile of the P-CSCF registered in NRF shall include the IP address, the supported transport type (TCP, UDP or both), the corresponding port number with priority indication for each transport type and may include an FQDN if available. The same applies to the Nnrf_NFManagement_NFUpdate Request.

NOTE:	The network sets the transport priority based on its preference and considering its specific deployment.

Based on the same configuration, a P-CSCF taken out of service will deregister itself using the Nnrf_NFManagement_NFDeregister Request.

*************** End of changes ***************