3GPP TSG-SA WG2 Meeting #135 	S2-1909147

Split, Croatia, October 14-18	(revision of S2-196965)

Source:	Ericsson

Title:	Discussion related to binding when consumer later becomes producer

Document for:	Agreement

Agenda Item: 	6.19.2

Work Item / Release:	5G_eSBA /Rel-16

Abstract of the contribution: This contribution discusses the use case where a consumer may become a producer

# 1	Discussion

Clause 4.17.11.3 “Binding created as part of service request” in 23.502 describes a theoretical case where a NF service consumer may later become a NF service producer and therefore it may provide “in advance” a binding indication valid for the offered NF Service producer. To our knowledge it is only one NF that behaves like this, and that is the AMF.

Observation 1: The case described in clause 4.17.11.3 in 23.502 is only applicable when AMF is the NF consumer that later becomes NF producer.

For example. UDM, SMSF, PCF and SMF first get a request to one of their services and later on they may become service consumers of AMF service. See excerpt from 23.502 below.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Request/ Response | Peer AMF |
|  | CreateUEContext | Request/ Response | Peer AMF |
|  | ReleaseUEContext | Request/ Response | Peer AMF |
|  | RegistrationCompleteNotify | Subscribe / Notify | Peer AMF |
|  | N1MessageNotify | Subscribe / Notify | SMF, SMSF, PCF, LMF, Peer AMF |
|  | N1MessageSubscribe |  | SMF, SMSF, PCF |
|  | N1MessageUnSubscribe |  | SMF, SMSF, PCF |
|  | N1N2MessageTransfer | Request/ Response | SMF, SMSF, PCF, LMF |
|  | N1N2TransferFailureNotification | Subscribe / Notify | SMF, SMSF, PCF, LMF |
|  | N2InfoSubscribe | Subscribe / Notify | NOTE 1 |
|  | N2InfoUnSubscribe |  | NOTE 1 |
|  | N2InfoNotify |  | AMF, LMF |
|  | EBIAssignment | Request/Response | SMF |
|  | AMFStatusChangeSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeUnSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeNotify | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
| Namf_EventExposure | Subscribe | Subscribe / Notify | NEF, SMF, UDM |
|  | Unsubscribe | Subscribe / Notify | NEF, SMF, UDM |
|  | Notify | Subscribe / Notify | NEF, SMF, UDM |
| Namf_MT | EnableUEReachability | Request/Response | SMSF |
|  | ProvideDomainSelectionInfo | Request/Response | UDM |
| Namf_Location | ProvidePositioningInfo | Request/Response | GMLC |
|  | EventNotify | Subscribe / Notify | GMLC |
|  | ProvideLocationInfo | Request/Response | UDM |
| NOTE 1: In this Release of the specification no known consumer is identified to use this service operation. |  |  |  |

UDM may use all services provided by AMF. So, if a binding indication is received from AMF to UDM, it is not clear to which service the binding would apply to. For example, binding to NF service set, it may be so that not all AMF services are within a NF service set!

Observation 2: It is not clear to what service the binding indication relates to for the case described in clause 4.17.11.3 in 23.502, since the consumer (the AMF) may expose several services

Proposal: The binding information shall not be used in request for which a NF service consumer may become a NF service producer.

# 2	Proposal

It is proposed to agree on proposal and approve accompanied CR 1470 in S2-1909148