3GPP TSG-WG SA2 Meeting #135 	S2-1909272

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0067 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NF load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 23.288 Clause 6.5, UPF can be the target NF type for NF load analytics. However there is NOTE that the list of possible instances is limited to AMF, N3IWF, SMF and PCF. There is no NF type or other parameters in Analytics Filter Information that can support the consumer to request UPF load analytics. It is proposed to add NF type and S-NSSAI as optional parameters in Analytics Filter Information in order to facilitate the consumer to request UPF load analytics. AF (e.g. V2X server) may also consume the service to get UPF load analytics. So it is proposed to add AF in the possible service consumer list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the note. Add S-NSSAI and NF type as optional parameters in Analytics Filter Information. Adding AF as the possible service consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s an inconvenience for the consumer which cannot provide the list of NF Instance IDs easily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.5	NF load analytics

6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF (e.g. AMF, SMF, NSSF, AF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "NF load information";

-	The Target of Analytics Reporting;

-	Analytics Filter Information:

- List of Instance IDs (optional);

- Target NF type (optional);

- S-NSSAI (optional).

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If the NF Instance IDs are provided, the NWDAF shall provide the analytics for each designated NF instance. In such case the Target of Analytics Reporting should be ignored. Otherwise, the NWDAF shall use the specific UE ID to determine which instances (AMF, N3IWF, SMF and PCF) are serving this specific UE.

NOTE:	The list of possible instances serving this specific UE is limited to AMF, N3IWF, SMF and PCF.

If the List of Instance IDs are provided, the Target NF type and S-NSSAI should be ignored. Otherwise, NWDAF shall determine NF instance IDs which match Target NF type or S-NSSAI or both according to the data collection from NRF or OAM.

# * * * * End of changes * * * *