SA WG2 Meeting #135	S2-1909288

Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

Source:	Huawei, HiSilicon

Title:	Key issue on user consent of data collection and data analytic

Document for:	Discussion / Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes to a key issue on user consent of data collection and data analytics, this key issue is either related to the inter PLMN Roaming and inter Region Roaming, but also related to the legacy KI of R16 that UE driven analytics.

- Discussion

In R16, standards has already specified operator can collect network data and information and do analytics base on it. However, user’s data privacy and user consent for retrieving its related network data or AF data is not specified. For example there is a legacy editor note extracted from R16 TS 23.288:  User consent for retrieving user data in AF via NEF is not specified in this Release. Similarly user consent for retrieving the network data is not specified either.

In the inter-PLMN roaming or inter-Region roaming scenario, which is an objective and work task of the new SID FS_eNA_Ph2, there is a strict restriction for data collection due to user data privacy protection, it is difficult to collect data generated on terminals without user consent, e.g. GPS information, terminal user input habits, etc. And it is also difficult to collect user-level data generated on the network side without user consent. In addition, for UE is required to report data authorization information, this issue may be related to legacy KI of R16 that UE driven analytics.

So in R17, it is necessary to discuss and specify that how to obtain user’s authorization for its related data in terminal and/or in application and/or in network side before data collection by NWDAF.

//Extract from SP-190557 for approved eNA phase2 SID//

- Objective 1: objective is to study the potential enhancements and new scenarios for network data analytics, including:

  - The remaining key issues from R16

    - KI#11: NWDAF-Assisted predictable network performance

Note: The study will consider any V2X solution defined as part of eV2XARC to possibly make it generic, if needed.

    - KI#13: UE driven analytics

Note: The method of collection of Data from UE for KI 13 will be investigated after MDT work of TSG RAN is completed for RAN impacting parameters.

………

  - Study whether we need the following functionalities and derive the related architecture enhancement to support:

    - Multiple NWDAF Instances in one PLMN including hierarchies, roles and inter-NWDAF instance cooperation

    - Inter PLMN Roaming and Inter Region Roaming

This paper is to propose a KI for FS_eNA_Ph2 that how to obtain user’s authorization for its related data in terminal and/or in application and/or in network side before data collection by NWDAF.

2.	Proposal

It is propose to include the KI into the TR 23.700-91.

* * * * First Change * * * *

5.X.1	KI X: <User data authorization>

5.X.1.1	Description

In R16, standards has already specified operator can collect network data and information and do analytics based on it. However, user’s data privacy and user consent for retrieving its related network data or AF data is not specified. For example there is a legacy editor note extracted from R16 TS 23.288:  User consent for retrieving user data in AF via NEF is not specified in this Release. Similarly user consent for retrieving the network data is not specified either.

In the inter-PLMN roaming or inter-Region roaming scenario and UE driven analytics scenario, due to user data privacy protection, it is difficult to collect data generated on terminals without user consent, e.g. GPS information etc. And it is also difficult to collect user-level data generated on the network side without user consent.

In this key issue, it is to discuss and specify that how to obtain user’s authorization for its related data in terminal and/or in application and/or in network side before data collection by NWDAF.

* * * * End of change * * * *