SA WG2 Meeting #135	S2-1909299

# 14 - 18 Oct 2019, Split, HR														 (revision of S2-19xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0024 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to delivery of PC5 QoS parameters to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PC5 QoS parameters to NG-RAN is provided from PCF to the AMF, the AMF stores such information as part of the UE context and provides those parameters to NG-RAN using NGAP. In the current solution, UE Configuration Update procedure is use for the purpose; however, UE Configuration Update procedure is to update UE configuration, in which the UE policy is encapsulated in a container to be delivered to the UE. Additionally, in UE Configuration Update procedure, PCF provides the PC5 QoS parameters to NG-RAN in the Namf_Communication_N1N2MessageTransfer message, which implies that the information is transparent to AMF. However, AMF needs to store it as the UE context. There’s no existing 23.502 procedures that can be reused directly for delivering PC5 QoS parameters to NG-RAN, thus it is proposed to document that in a new procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure of delivering PC5 QoS parameters to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong procedure used for delivering PC5 QoS parameters to NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.5.7	Delivery of PC5 QoS parameters to NG-RAN

Figure 6.5.7-1 describes the procedure of Delivery of PC5 QoS parameters to NG-RAN.

Figure 6.5.7-1: Delivery of PC5 QoS parameters to NG-RAN

This procedure concerns both roaming and non-roaming scenarios.

In the non-roaming case the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF:

1.	Step 1-4 of UE Policy Association Establishment Procedure applies with the following additions:

-	When receiving Npcf_UEPolicyControl Create Request from the AMF which includes a V2X capability indicator, PCF generates the PC5 QoS parameters as defined in clause 5.4.2 used by NG-RAN.

-	In the roaming case, the H-PCF includes the PC5 QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message sent to V-PCF, and V-PCF forwards the information to the AMF in Npcf_UEPolicyControl_Create Response In non-roaming case, the V-PCF includes the PC5 QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message sent to AMF..

The AMF stores such information as part of the UE context.

2. The AMF includes the PC5 QoS parameters in the NGAP message to the NG-RAN.

3. NG-RAN installs the received policy and reply with the result.

4. If the AMF receives the NG-RAN PC5 QoS parameters delivery result and the PCF subscribed to be notified of the reception of the NG-RAN PC5 QoS parameters delivery result, then the AMF forwards the result to the (V-) PCF using Namf_N2InfoNotify.

5. If the V-PCF receives notification of the reception of the NG-RAN PC5 QoS parameters, then the V-PCF forwards the notification response of the NG-RAN to the H-PCF using Npcf_UEPolicyControl_Update Request.

6.	The H-PCF sends a response to the V-PCF.

END OF CHANGES