SA WG2 Meeting #S2-135	S2-1909397

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1803 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Port model of the 5G bridge |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FFS on whether LLDP is supported in the DS-TT. TSN Translator should avoid non-TSN functionality as described in S2-1909396 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate TSN specific and non-TSN specific Ethernet port functionality, and use an ETHPort logical entity for the latter, residing in the UPF. Clarify that LLD is not supported in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LLDP support in UE would remain unclear. Non-TSN specific functionality would be required in TSN translators. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.2, 5.28.1, 5.28.3.1, 5.28.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 1802 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 4.4.8.2	Architecture to support Time Sensitive Communication

The 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5GS TSN translator functionality consists of Device-side TSN translator (DS-TT) and Network-side TSN translator (NW-TT). 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as any other TSN Bridge, the 5GS provides TSN ingress and egress ports via DS-TT and NW-TT for TSN specific functionality. Additionally, the 5GS provides a logical ETHPort residing in the UPF to support Ethernet port functionality that is not TSN specific (see Figure 4.4.8.2-2). DS-TT and NW-TT optionally supports the following functionalities:

-	Hold and forward functionality for the purpose of de-jittering;

The ETHPort optionally supports the following functionalities.

-	Link layer connectivity discovery and reporting as defined in IEEE 802.1AB [97].

There are three TSN configuration models defined in IEEE P802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this release of the specification;

-	fully distributed model is not supported in this release of the specification;

-	hybrid model is not supported in this Release of the specification.

NOTE 1:	This release only supports interworking with TSN using IEEE 802.1Qbv [96] based QoS scheduling.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

NOTE 2:	Whether DS-TT and UE are combined or are separate is up to implementation.

Figure 4.4.8.2-2: User plane view with ETHPort added for non-TSN related Ethernet port functionality

* * * * Next Change * * * *

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge has ports on the network side on a single UPF (i.e. PSA), and ports on the device side that provide connectivity to the terminal device  using the user plane tunnel between the UE and UPF. For each 5GS Bridge of a TSN network, the ports on the network side support the connectivity to the TSN network over the network interfaces of the UPF, the ports on device side are associated to the PDU Session providing connectivity to the TSN network.

The 5GS provides a logical ETHPort residing in the UPF to support Ethernet port functionality that is not TSN specific. One ETHPort is associated with each port of the UPF on the network side, and one ETHPort is associated in the UPF with each PDU Session. The TSN specific port functionality is provided by NW-TT on the network side, and DS-TT which resides on the UE.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on the device side, and a PDU Session during reporting of logical TSN bridge information.

All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on device side and network side are integrated as part of the configuration of the 5G virtual bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes the following:

-	Bridge ID of 5GS Bridge:

-	MAC address of the bridge;

-	bridge name;

-	number of ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97].

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Identity of the 5GS Bridge and port numbers of the network side ETHPorts could be preconfigured on the UPF, which is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port numbers of the device side ETHPortsassociated with the PDU Sessions are assigned by the UPF during PDU session establishment and stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released then SMF informs TSN AF accordingly.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

* * * * Next Change * * * *

### 5.28.3	Port management information exchange in 5GS

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in ETHPorts, DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and ETHPorts, DS-TT or NW-TT, respectively inside a Port Management Information Container. Table 5.28.3.1-1 lists standardized port management information.

Table 5.28.3.1-1: Standardized port management information

| Port management information |  | Applicability |  |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ETHPort | DS-TT | NW-TT |  |  |
| General |  |  |  |  |  |  |
| Port management capabilities (see Note 2) |  | X | X | X | R |  |
| Bridge delay related information |  |  |  |  |  |  |
| txPropagationDelay |  |  | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |  |  |
| Traffic class table |  |  | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |  |  |
| GateEnabled |  |  | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminBaseTime |  |  | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlList |  |  | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminCycleTime (see Note 3) |  |  | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlListLength (see Note 3) |  |  | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| Tick granularity |  |  | X | X | R | IEEE 802.1Qbv [96] Table 12-28 |
| ETHPort port neighbor discovery configuration |  |  |  |  |  |  |
| adminStatus |  | X |  |  | R | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortIdSubtype |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of ETHPort |  |  |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X |  |  | R | IEEE 802.1AB [97] Table 11-2 |
|  | NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. |  |  |  |  |  |

Editor's note:	Whether TSN AF can write DS-TT and NW-TT port neighbour discovery configuration is FFS.

Exchange of port management information is initiated by TSN AF to:

1)	get port management capabilities, i.e. read information indicating which standardized and deployment-specific port management information is supported by ETHPorts, DS-TT or NW-TT;

2)	get port management information for an ETHPort, DS-TT or NW-TT;

3)	set port management information for an ETHPort, DS-TT or NW-TT;

4)	subscribe to notifications if specific port management information for an ETHPort, DS-TT or NW-TT port changes.

Exchange of port management information is initiated by ETHPort, DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

TSN AF indicates inside the Port Management Information Container whether it wants to get or set port management information or intends to subscribe for notifications.

Editor's note:	Whether TSN AF needs to be able to read port management information and needs to be able to get port management capabilities is FFS. This relates to bullets 1, 2, 4 and the sentence above.

* * * * Next Change * * * *

#### 5.28.3.2	Transfer of port management information

Port management information is transferred transparently via 5GS between TSN AF and ETHPorts, DS-TT or NW-TT, respectively, inside a Port Management Information Container as follows:

-	To convey port management information from an ETHPort, DS-TT or NW-TT to TSN AF:

-	DS-TT provides a Port Management Information Container to the UE, which includes the Port Management Information container as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Modification procedure to forward the Port Management Information container to the SMF. SMF forwards the Port Management Information container and the port number of the related DS-TT Ethernet port to TSN AF as described in TS 23.502 [3] clause 4.3.3.2;

-	NW-TT and ETHPort provides a Port Management Information Container to the UPF, which triggers the N4 Session Level Reporting Procedure to forward the Port Management Information Container to SMF. SMF in turn forwards the container and the port number of the related NW-TT/ETHPort to TSN AF as described in TS 23.502 [3] clause 4.16.5.1. If NW-TT/ETHPort on the network side provides a Port Management Information Container, then UPF forwards the Port Management Information Container only for one N4 session. In case 5G VN is in use, the NW-TT/ETHPort on the network side may provide the Port Management Information Container using the group-level N4 session corresponding to the given 5G-VN.

-	To convey port management information from TSN AF to ETHPort, DS-TT or NW-TT:

-	TSN AF separates TSN specific and non-TSN specific port management information. The TSN AF provides a Port Management Information Container, MAC address reported for a PDU Session and the port number of the Ethernet port to manage, as well as an indication whether the information is TSN specific to the PCF, which forwards the information to SMF based on the MAC address using the PCF initiated SM Policy Association Modification procedure as described in TS 23.502 [3] clause 4.16.5.2. Based on the indication whether the information is TSN specific, the SMF determines whether to forward the information to an ETHPort (non TSN specific information) or to a DS-TT or NW-TT (TSN specific information). In case of TSN specific information SMF determines whether the port number relates to a DS-TT or NW-TT. The SMF forwards the Port Management Information Container to DS-TT,NW-TT or ETHPort using the network requested PDU Session Modification procedure as described in TS 23.502 [3] clause 4.3.3.2. If the port number identifies an NW-TT/ETHPort on the network side, PCF and SMF forward the Port Management Information Container only for one PDU session. In case 5G VN is in use, the SMF may forward the Port Management Information Container to an NW-TT/ETHPort on the network side using the group-level N4 session corresponding to the given 5G-VN.

* * * * End of Change * * * *