SA WG2 Meeting #135 	S2-1909513

14 - 18 Oct 2019, Split, Croatia	 (revision of S2-190xxxx)

Source:	CATT

Title:	New KI: Support of Efficient Information Provision to Application Server

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: this paper proposes to add a new key issue on how to efficiently provide application server with information from network.

Discussion

In order to improve user experience, application servers can make changes on transport or application layer based on information provided by 5G network. For example, the video application can change video quality based on the QoS of data path to avoid video streaming’s slow buffering. So what information and how to efficiently provide information to application server should also be considered.

This P-CR is proposed to add a new key issue about the questions above.

Proposal

It is proposed to add a new key issue as following: (all new text).

FIRST CHANGE

5.x	Key Issue x: Support of Efficient Information Provision to Application Server

5.x.1	Description

In order to improve user experience, application servers can make changes on transport or application layer based on information provided by 5G network. For example, the video application can change video quality based on the QoS of data path to avoid video streaming’s slow buffering. So what information and how to efficiently provide information to application server should also be considered.

This key issue will study:

-	What information can be provided to application servers?

-	Which and how does the network function decide to provide information to application server? Based on what kind of criteria? Whether PCF is involved to decide the criteria?

-	The path to transmit the information efficiently to application server.

End of CHANGE