3GPP TSG-SA WG2 Meeting #135	S2-1909813

14 - 18 October, 2019, Split Croatia	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1903 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of IAB operation in EN-DC mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As documented in TS 23.501 clause 5.35.1, the IAB operation support both SA mode and EN-DC mode. For EN-DC mode, there were three different secenarios identified by RAN WGs, i.e. both UE and IAB-node connect to EPC; UE connects to EPC, but IAB-node connects to 5GC; and UE connects to 5GC, but IAB-node connects to EPC. The first scenario conerns only EPC and would be documented in TS 23.401. The latter two cases concer also 5GC, and therefore should be documented in TS 23.501. Therefore, it is proposed to add the corresponding description of the two IAB operation mode in a new clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new clause on IAB operation in EN-DC mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB operation with EN-DC mode is not supported in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.1; (new) 5.35.x; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.35.1	IAB architecture and functional entities

Integrated access and backhaul (IAB) enables wireless in-band and out-of-band relaying of NR Uu access traffic via NR Uu backhaul links.

At high level, IAB has the following characteristics:

-	IAB uses the CU/DU architecture defined in TS 38.401 [42], and the IAB operation via F1 (between IAB-donor and IAB-node) is invisible to the 5GC;

-	IAB performs relaying at layer-2, and therefore does not require a local UPF;

-	IAB supports multi-hop backhauling;

-	IAB supports dynamic topology update, i.e. the IAB-node can change the parent node, e.g. another IAB-node, or the IAB-donor, during operation, for example in response to backhaul link failure or blockage.

Figure 5.35.1-1 shows the IAB reference architecture with two backhaul hops, when connected to 5GC.

Figure 5.35.1-1: IAB architecture for 5GS

Editor's note:	This figure may be updated based on progress on IAB in RAN WGs.

Each relay, referred to as IAB-node, consists of a gNB-DU function and a UE function (referred to as MT). The gNB-DU in the IAB-node is responsible for providing NR Uu access to UEs and child IAB-nodes. The corresponding gNB-CU function resides on the IAB-donor gNB, which controls IAB-node gNB-DU via the F1 interface. IAB-node appears as a normal gNB to UEs and other IAB-nodes, and allows them to connect to the 5GC.

Editor's note:	RAN WGs chose to refer to the UE function in the IAB-node as MT. This may cause confusion and SA2 will determine a more suitable name.

The IAB-node MT function behaves as a UE, and reuses UE procedures to connect to:

-	the gNB-DU on a parent IAB-node or IAB-donor for access and backhauling;

-	the gNB-CU on the IAB-donor via RRC for control of the access and backhaul link;

-	5GC, e.g. AMF, via NAS;

-	OAM system via a PDU session or PDN connection (based on implementation).

Editor's note:	It is FFS if new 5QI/QCI needs to be defined for the OAM traffic.

The IAB-node can connect to 5GC over NR (SA mode) or connect to EPC (EN-DC mode). The UE connecting to the IAB-node can operate in the same or different modes than the IAB-node as defined in TS 38.401 [42]. The architecture with both UE and IAB-node connected to EPC is covered in TS 23.401 [26]. The architectures with UE and IAB-node operates in different modes are provided in clause 5.35.x.

*** Next CHANGE ***

### 5.35.x	Support of IAB with EPC interworking

IAB operation also supports the case where IAB-donor gNB has connections to both EPC and 5GC. Based on PLMN configuration, there are two different operation modes.

Figure 5.35.x-1 shows the IAB reference architecture with two backhaul hops, where the IAB-node connects to a 5GC via NR, and the UE connects to the EPC with Dual Connectivity as defined in TS 37.340 [85]. In this operation mode, the 5GC is restricted for IAB-UE to access only.

Figure 5.35.x-1: IAB architecture with IAB-node connected to 5GC and UE connected to EPC

Figure 5.35.x-2 shows the IAB reference architecture with two backhaul hops, where the IAB-node connects to the EPC with Dual Connectivity as defined in TS 37.340 [85], and the UE connects to the 5GC via NR Uu. In this operation mode, the EPC is restricted for IAB-UE access only.

Figure 5.35.x-2: IAB architecture with IAB-node connected to EPC and UE connected to 5GC

To support the above operation modes, the IAB-UE shall be configured to select only a specific PLMN (as defined in TS 23.122 [17]) and whether it needs to connect to 5GC or EPC.

NOTE: For a particular PLMN, it is expected that only one of the modes would be deployed in a known region.

*** END CHANGES ***