3GPP TSG-SA2 Meeting #135	S2-1909990

# Split, Croatia，14 - 18 October 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0019 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supplement for address resolution and clarification for Localized V2X Application Server discovery and routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.1 |  |  |  |  |  | Date: |  |  | 2019-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This paper proposes to supplement DNS address resolution and clarification for localized V2X Application Server discovery and routing. The first change is that, besides the UE configured to perform DNS resolution when it crosses configured geographic areas, the V2X application server may also trigger UE from application layer to perform DNS resolution based on SMF notification. In 5.3.2, the last sentence UPF is responsible for routing the traffic to the appropriate local V2X Application Servers based on Anycast address is not clear for which UPF should be responsible for the local V2X application server. To support the local V2X application server discovery and traffic routing, the local UPF should be selected based on AF influence mechanism as in Annex A. Otherwise, the traffic can’t be transfer to the appropriate local V2X Application Server The V2X Application Server can send AF request including its location information (e.g. DNAI) to SMF. SMF will choose a local UPF based on UE location and DNAI, and then the local UPF can route the traffic to the appropriate local V2X Application Server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | V2X application server may also trigger UE from application layer to perform DNS resolution based on SMF notification. A local UPF needs to be selected by SMF based on the AF influence request from localized V2X Application Servers, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear of the current specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1,5.3.2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.3.1	General

A UE needs to discover the V2X Application Server(s), when V2X communication over Uu operation mode is used. The V2X Application Server address information may be configured on the UE or provisioned over N1 reference point, as specified in clause 5.1.3.1.

When the configuration contains the FQDN(s), the UE shall perform DNS to resolve the address(es) of the V2X Application Server. The UE may use the configured V2X Application Server information only in the designated geographical area. When the UE changes serving PLMN or crosses configured geographic areas, it should perform address resolution again.

NOTE: When the V2X Application Server is notified by SMF on UE change of user plane path, as specified in clause 5.6.7 of TS 23.501[6], it may trigger the UE from application layer to perform DNS address resolution.

5.3.2	Multiple V2X Application Server and Localized V2X Application Server discovery and routing

Multiple V2X Application Servers may be involved in the V2X communication, each providing particular V2X services and/or serving a particular geographical region. Therefore, the V2X Application Server address information as specified in clause 5.1.3.1 can contain multiple servers' information. When multiple V2X Application Servers are configured, the application layer will choose the proper V2X Application Server to use.

When localized V2X Application Servers are deployed, Anycast may be used to conceal the server change from the UE. In this case, a FQDN is configured for a large region, e.g. the entire PLMN, and the UE only needs to resolve it once to an Anycast address. The UPF is responsible for routing the traffic to the appropriate local V2X Application Servers based on Anycast address.

NOTE: When Application Function influence on traffic routing, as defined in clause 5.6.7 of TS 23.501[6], is applied, a local UPF is selected to route the traffic to the local V2X application server.

* * * * End of Changes * * * *

Annex A (informative):
Application Function influence based edge computing traffic routing for V2X services

To route V2X messages or any traffic for V2X purposes to/from V2X Application Server(s) in a local Data Network (DN) close to NG-RAN, V2X Application Server operated by e.g. operators, OEMs, road authorities, can use the Application Function (AF) influence on traffic routing feature defined in clause 5.6.7 of TS 23.501 [6].

The V2X Application Server that sends the AF request for AF influence based edge computing for V2X services can be the same V2X Application Server to/from which the traffic is routed.

If SMF provides to the UPF the traffic steering policy corresponding to the DNAI generated based on the AF input, the UPF shall route the traffic to the appropriate local V2X Application Server based on the traffic steering policy. In that case, the V2X Application Server discovery is unnecessary.