SA WG2 Meeting #135 	S2-1910278

# October 14-18th, 2019 ; Split, HR		                               		  (revision of S2-1908768)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1698 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to ETSUN specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clean up, clarifications are needed, see summary of changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3.1 Definitions: simplify the definition of I-SMF 5.34.3 I-SMF selection: clarifications on the AMF algorithm and on the SMF selection in case of delegated discovery 5.34.2.X: N16a is based on N16 but with additions defined in clause 5.34 and especially in clause 5.34.4 to 5.34.6 for the case of traffic offloading involving an I-SMF. 5.34.7: In the case of I-SMF change or removal, the AMF considers the I-SMF subscription to AMF mobility event as cancelled; the new I-SMF needs to initiate a subscription to the AMF events where needed. Subscriptions from SMF to AMF events remain as before the I-SMF insertion, change or removal This is because - In case of I-SMF removal, the I-SMF subscription is now obolete - anyhow In case of I-SMF change the new I-SMF and the old I-SMF are likely not in the same set), thus the new I-SMF would need to change the Notification Target Address (+ Notification Correlation ID) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 5.34.3 ;; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev1 - Definition changed - Removed new clause on N16a - Removed change on 5.34.7 |
| --- | --- |

FIRST CHANGE

## 3.1	Definitions

[…]

Intermediate SMF (I-SMF): An SMF that is inserted, to support a PDU session as the UE is located in an area which cannot be controlled by the original SMF because the UPF(s) belong to a different SMF Service Area.

NEXT CHANGE (2)

### 5.34.3	I-SMF selection

The AMF is responsible of detecting when to add or to remove an I-SMF for a PDU Session. For this purpose the AMF gets from NRF information about the Service Area of SMF(s). During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI. Conversely if the AMF detects that an I-SMF is no more needed (as the service area of the SMF includes the new UE location) it removes the I-SMF and interfaces directly with the SMF of the PDU Session. If the AMF detects that the SMF cannot serve the UE location (e.g. due to mobility), then the AMF selects a new I-SMF serving the UE location. If the existing I-SMF cannot serve the UE location and the service area of the SMF does not include the new UE location, then the AMF initiates an I-SMF change.

At PDU Session Establishment in non-roaming and roaming with LBO scenarios, if the AMF or SCP cannot select an SMF with a Service Area supporting the current UE location for the selected DNN, S-NSSAI and required SMF capabilities, the AMF selects an SMF for the selected DNN, S-NSSAI and required capabilities and in addition selects an I-SMF serving the UE location. During the lifetime of the PDU Session, the AMF selects a new I-SMF if it determines that neither the current SMF nor the current I-SMF (if any) can serve the UE location.

Comparing to the SMF selection function defined in clause 6.3.2, the following parameters are not applicable for I-SMF selection:

-	Data Network Name (DNN).

-	Subscription information from UDM.

NOTE:	All SMF(s) and I-SMF are assumed to be able to control the UPF mapping between EPC bearers and 5GC QoS flows.

If delegated discovery is used :

-	The AMF sends Nsmf_PDUSession_CreateSMContext Request to SCP and includes the parameters as defined in clause 6.3.2 (e.g. the DNN, required SMF capabilities, UE location) as discovery and selection parameters. If the SCP successfully selects an SMF matching all discovery and selection parameters, the SCP forwards the Nsmf_PDUSessionCreateSMContext Request to the selected SMF.

-	If the SCP cannot select an SMF matching all discovery and selection parameters, the SCP returns a dedicated error to AMF. In this case the I-SMF also need be discovered.

-	Upon reception of the error from the SCP due to the SMF matching all discovery and selection parameters cannot be found, the AMF performs the discovery and selection of the SMF from NRF (thus not providing the UE location as a discovery parameter). The AMF may indicate the maximum number of SMF instances to be returned from NRF, i.e. SMF selection at NRF.

-	The AMF sends Nsmf_PDUSession_CreateSMContext Request to SCP, which includes the selected SMF ID and the discovery and selection parameters as defined in clause 6.3.2 except the DNN and the required SMF capabilities, i.e. parameter for I-SMF selection. The SCP performs discovery and selection of the I-SMF and forwards the Nsmf_PDUSession_CreateSMContext Request to the selected I-SMF.

-	The I-SMF sends the Nsmf_PDUSession_Create Request towards the SMF via the SCP, which includes the received SMF ID as a selection parameter. The SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the SMF.

-	The SMF answers to the I-SMF that answers to the AMF; in this answer the AMF receives the I-SMF ID

-	Upon reception of a response from I-SMF, based on the received I-SMF ID, the AMF may obtain the SMF Service Area of the I-SMF from NRF. The AMF uses the SMF Service Area of the I-SMF to determine the need for I-SMF relocation upon subsequent UE mobility.

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES