3GPP TSG-SA WG2 Meeting #135	S2-1910289

# 14 - 18 October, 2019, Split Croatia

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1753 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ETSUN: Mobility event management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the mobility, both AMF and I-SMF may be changed. So the subscription information, e.g. subscription ID and notification information need be updated accordingly. For the subscription to AMF, i.e. mobility event: According to current description, when the AMF change, the AMF update the subscription ID stored at the I-SMF/SMF only in case of no I-SMF change/removal. So in case if AMF change and I-SMF change, the AMF will not notify the new subscription ID to the SMF/I-SMF respecitively. This is correct for the I-SMF as the new I-SMF trigger a new subscription. However this is incorrect for SMF as the SMF always need be notified of the new subscription ID in the new AMF. To simplify the AMF behaviour, if the AMF change, the AMF always notify the I-SMF/SMF the new subscription ID information per the notification information. If the I-SMF has been changed and the context in old I-SMF has been deleted, the update may fail with a response code “404 not found”, the AMF can delete the subscription in this case. If I-SMF change, per existing description,the new I-SMF may update the subscription or trigger a new subscription. However due to the AMF may also be reallocated, the subscription update only apply the case the AMF is not reallocated. A new subscription triggered from the new I-SMF apply to all case. To simplify the I-SMF behaviour, it is proposed only the new subscription activation is defined. I-SMF insertion case is similar as adding a new I-SMF without removal of old I-SMF. It can do in the similar way. In a short, the AMF do the subscription ID update if the AMF change. The SMF trigger a new subscription if the SMF change. For the subscription to SMF: The AMF may explicitly subscribe to exposure services provided by I-SMF.The similar handling is suggested. For the implicit subscription requested by AMF, the AMF can always update the SMF with the new notification information. In all case for the subscription from the old NF, the subscription can be implicitly removed after the subscription timer expired per TS29.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the target AMF update the subscription with I-SMF if AMF change. And the I-SMF trigger a new subscription if a new I-SMF is changed. Also how to do the SMF subscription from AMF is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is complexity for the SMF/AMF to handle the subscription ID /notification information change in all case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.7, 5.34.7.1(new), 5.34.7.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

5.34.7	Event Management

5.34.7.1	UE's Mobility Event Management

When an I-SMF is involved in a PDU Session, the SMF and I-SMF independently subscribes to "UE mobility event notification" service provided by AMF. The AMF treats the SMF's and I-SMF's subscription separately and notifies the event directly to the SMF and I-SMF. If the SMF does not know the serving AMF address, the SMF gets the serving AMF address from the UDM as described in clause 5.2.3.2.4, TS 23.502 [3] and subscribes directly with the serving AMF.

In the case of AMF change (e.g. Inter NG-RAN node N2 based handover), the target AMF receives mobility event information from the source AMF and update the mobility event subscription information with the SMF and I-SMF independently (i.e. target AMF allocates the Subscription Correlation ID for each event and notify the respective SMFs and I-SMF as described in clause 5.3.4.4).

In the case of I-SMF change or I-SMF insertion (e.g. Inter NG-RAN node N2 based handover), the subscription of mobility event is transferred from the old I-SMF or SMF to the new I-SMF respectively, the new I-SMF triggers a new subscription event if the new I-SMF wants to maintain the event (e.g. due to an AF event subscription dependence). In case of I-SMF removal, the subscription of mobility event is transferred from the old I-SMF to SMF, the SMF triggers a new subscription event if the SMF wants to maintain the event.

NOTE: 	The subscription from the old NF can be implicitly removed after the subscription timer expires.

2nd CHANGE

5.34.7.2	SMF Event Management

The AMF may explicitly subscribe an event notification provided by SMF. In case of I-SMF insertion/change, the new I-SMF receives the subscription of AMF from the SMF or old I-SMF respectively, the new I-SMF update the subscription information with the AMF. In case of I-SMF removal, the SMF receives the subscription of AMF from the old I-SMF, the SMF update the subscription information with the AMF. In case of AMF change, the subscription event is transferred to the new AMF, the new AMF triggers new subscription event if the new AMF wants to maintain the event.

The AMF implicitly subscribe an event notification provided by SMF, e.g. SM context status notification. In case of I-SMF insertion/change, when the AMF update the signalling path with the new I-SMF, the AMF provides a new notification information to new I-SMF. In case of I-SMF removal, when the AMF update the signalling path with the SMF, the AMF provides a new notification information to SMF.

END OF CHANGES