3GPP TSG-SA WG2 Meeting #135	S2-1910364

# Split, Croatia, October 14–18, 2019 	(revision of S2-1910251)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0049 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of IP address allocation for FN-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The chart described in Figure 7.2.1.3-1 for FN-RG Registration via W-5GAN is unclear. The IP address allocation mechanism for a FN-RG is not only DHCP-based, but also can be IPCP/IPv6CP-based. IP address request initiated by FN-RG canbe PPPoE based and IPoE based. In case of PPPoE, as W-AGF cannot deal with IPCP/IPv6CP message and cannot ask for deferred IP address allocation, the W-AGF will cache the IP address request until the 5GC completes the PDU session establishment and sends back the IP address obtained from 5GC to FN-RG. In case of IPoE, W-AGF can ask for deferred IP address allocation - If asked and 5GC accepts, W-AGF will send the received DHCP request beforehand from FN-RG to 5GC to obtain the IP address via Deffered IP address allocation mechanism after Step 5. - Otherwise, W-AGF will do the same procedures as PPPoE situation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the deferred and non-deferred IP address allocation procedure between FN-RG and W-AGF in more detail. 1). Add the call flow between FN-RG and W-AGF as L2 message, which is similar with the L2 connection in FN-RG registration procedure in Step 0. The means of L2 connection is defined by BBF for FN-BRG and by CableLabs for FN-CRG. 2). Step 6 is divided into Step 6a and Step 6b. - Where step 6a is the the deferred IP address allocation request procedure proceeded by W-AGF via the established PDU Session User Plane. After this procedure, the W-AGF will obtain the IP address/prefix. - Step 6b is the response of IP address/prefix request corresponding to step 0. If it is non-deferred IP address allocation use case, the replying message will be the UE IP address received from 5GC via NAS; while in deferred IP address allcoation use case, the replying message will be the IP address received in step 6a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 7.3.4	FN-RG related PDU Session Establishment via W-5GAN

The procedure below is based on the PDU Session Establishment procedure specified in TS 23.502 [3] clause 4.3.2.2.1.

Figure 7.3.4-1: FN-RG related PDU Session Establishment via W-5GAN

- [Optional] FN-RG sends an IP address/prefix request to the W-AGF via the L2 connection established in clauses 7.2.1.3.

NOTE: 	This IP address/prefix request can also be sent by FN-RG later in this procedure; the W-AGF may store this and complete the address allocation via 5GC after the PDU session setup.  The means of carrying the IP address/prefix request/response between FN-BRG and W-AGF is defined in BBF WT-456 [9], and between FN-CRG and W-AGF is defined in CableLabes WR-TR-5WWC-ARCH [27].

1.	After the registration procedure is completed, the W-AGF may establish PDU session(s) on behalf of the FN-RG. The trigger for W-AGF to initiate a PDU establishment process is defined in BBF WT-456 [9] and CableLabs WR-TR-5WWC-ARCH [27].

The W-AGF generates a PDU session ID and derives the parameters for the PDU Session (PDU Session type, S-NSSAI, DNN, SSC mode, etc.) based on signalling received from the FN-RG (DHCP, IPv6 RS, …), on local configuration, and on information received from the 5GC (e.g. during the Registration procedure or when received URSP rules) and stored on the W-AGF.

If W-AGF has received a DHCPv4/DHCPv6 request from the FN-RG, it may request a PDU Session with deferred IP address allocation.

The W-AGF sends a NAS PDU Establishment Request to the AMF. This request contains the PDU Session ID, and may contain a Requested PDU Session Type, a Requested SSC mode, 5GSM Capability PCO, SM PDU DN Request Container, Number of Packet Filters.

The W-AGF sends NAS PDU Establishment Request in a N2 Uplink NAS transport message (NAS message, User location information, W-AGF identities).

The W-AGF identities contains a list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF and can be used by SMF in step 8 in TS 23.502 [3] clause 4.3.2.2.1 as input to select an UPF.

2a.	The PDU session request is processed in the 5GC as per steps 2-11 of TS 23.502 [3] clause 4.3.2.2.1. These steps are for UPF selection and resource reservation/allocation in the UPF. With regard to TS 23.502 [3], an additional parameter is sent from AMF to SMF i.e. the list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF received by the the AMF from the W-AGF. The SMF can use it in step 8 for UPF selection as per clause 4.3.2.2.1.

2b.	The SMF responds via AMF as defined in step 11 of clause 4.3.2.2.1 in TS 23.502 [3] with an N2 PDU Session Request that includes QoS profile(s), PDU Session ID, PDU Session Establishment Accept and the N3 tunnel endpoint information for the UPF. The differences with step 11/12 of TS 23.502 [3] clause 4.3.2.2.1 are

-	The W-AGF shall ignore RSN if received from 5GC.

3.	Based on its own policies, configuration and based on the QoS flows, QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session.

The W-AGF may, as defined in BBF WT-456 [9] and CableLabs WR-TR-5WWC-ARCH [27], perform Access specific resource reservation with the AN, that is, it sets up the W-UP resources for the PDU session.

4.	The W-AGF allocates AN N3 tunnel information for the PDU Session and includes the AN N3 tunnel endpoint information in the N2 PDU Session Setup Response message to the AMF.

5.	The PDU session setup procedure is completed in 5GC. All steps after step 13 as specified in TS 23.502 [3] figure 4.3.2.2.1 are executed.

6a.	If W-AGF requested deferred IP address allocation in step 1 and this was accepted by the network, the W-AGF sends on the user Plane of the PDU Session any DHCP or RS message received beforehand from the FN-RG to the 5GC to obtain the IP address/prefix.

6b. W-AGF completes the IP address/prefix allocation with the FN-RG via the established L2 connection. If W-AGF did not request deferred IP address allocation in step 1a, the IP address/prefix sent back to the FN-RG is the UE IP address/prefix delivered in NAS message in step 2b. If W-AGF requested deferred IP address allocation in step 1a, the IP address/prefix sent back to the FN-RG is the UE IP address/prefix delived via deffered IP address allocation procedures in step 6a.

**** End of Changes ****