3GPP TSG-WG SA2 Meeting #135 	S2-1910558

# Split, Croatia, October 14 – 18, 2019	(revision of S2-1910095/09271)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0066 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections/updates to TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Table 6.7.5.2-1, the two terminologies in the NOTE i.e "the UE behavioural information" used in the Abnormal classifier training phase and "the UE behavioural information" used in the Abnormal classifier inference phase are ambiguous. Some others Miscellaneous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Miscellaneous correction or update to TS 23.288 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear decriptions in TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.4.1, 6.4.4, 6.4.5, 6.7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Changes * * * *

6.4	Observed Service experience related network data analytics

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	global Service Experience (i.e. for all UEs) for a Network Slice, given an Application or a set of Applications in a Network Slice;

-	detailed Service Experience (i.e. a group of UEs or a list of UEs) for a given Application.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information: one or more combinations of the following Analytics Filters: Area of Interest, Media/application bandwidth, S-NSSAI, DNN, DNAI, Application ID;

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

WDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provides statistics and predictions on the observed service experience to a consumer NF or to OAM.

NOTE:	OAM uses the Observed Service Experience as defined in SA WG5 TSs.

6.4.2	Input Data

The service data collected from the AF and the network data from other 5GC NFs for observed service experience are defined in Table 6.4.2-1 and Table 6.4.2-2.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

* * * * Next Change * * * *

6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure to NWDAF assisted QoS profile provisioning and adjustment

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE".  At the same time, for an Application Id, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = Any UE) as defined TS 23.502 [3].

NOTE 1:	In the case of external AF, NEF translates the requested Area of Interest into a list of geographic zone identifier(s) as described in clause 5.6.7.1 of TS 23.501 [2].

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

NOTE 3:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 4:	The call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

If the consumer NF is PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 5:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

* * * * Next Change * * * *

6.4.5	Procedures to request Service Experience reported for UEs on a Network Slice

This procedure is similar to the procedure in clause 6.4.4.

The difference compared to the procedure in clause 6.4.4 is that it allows the consumer to request Analytics ID "Service Experience" for a list of SUPI(s) or Internal Group Identifier(s).

Therefore, Observed Service experience may be provided individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The Observed Service Experience for a network slice when consumed by OAM could be used as described in SA WG5 TS 28.550 [7] Annex H.

* * * * Next Change * * * *

6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information and/or expected UE behavioural parameters from the 5GC NFs (SMF, AMF, AF).

The UE behavioural information collected from 5GC NFs is as specified in clauses 6.7.2.2 and 6.7.3.2.

The expected UE behavioural parameters provided to the NWDAF are defined in clause 4.15.6.3, TS 23.502 [3].

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE) |  |
| >Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| NOTE: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

* * * * End of Changes * * * *