3GPP TSG-SA2 Meeting #135	S2-1910564

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019	(rev. of S2-1910098)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0002 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to NF load data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Impossibility to request the load of a subset of NFs which serve a UE - Impossibility to request easily the load an NF set - Missing NFs in the stated list of observable NFs: I-SMF, I-UPF, UPF - Inconvenience for the consumer to get analytics of NF in a slice - Unclear analytics on the NF status (registered, suspended, etc.) - An editor’s note on NF load analytics regarding NRF providing information on number of NF instances needs to be addressed. - Unclear definition of resource information to be retrieved from OAM and available serviced to be used from SA5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Possibility to request the load of a subset of NFs which serves a UE, by providing a list of types of targeted NFs (e.g. AMF, PCF) instead of instance IDs. - Possibility to request the load of an NF set - Add to list of possible NFs types: I-SMF, I-UPF, UDM and UPF - Add S-NSSAI as opitonal parameter in Analytics Filter Information - Add the types of instance in the output - Add analytics per value of NF status (registered, suspended, undiscoverable) - Minor additional editorial modifications are provided to enumerate aspects where OAM information could complement NRF information. - Various text rephrasing for clarification. - Alignment among clauses about the information retrieve from OAM - Add the NF resource configuration as input for NF Load Analytics - Remove FFS in Clause 6.5.4 defining the exact services in SA5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obligation for an application to know the instance IDs before requesting the NWDAF or for understanding of the outputs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.5.1, 6.5.2, 6.5.3, 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev1 is a merge of rev0 with CR0030 and CR0032. rev2 also merges CR0067. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.2.3.1	General

The NWDAF may collect relevant management data from the services in the OAM as configured by the PLMN operator.

-	NG RAN or 5GC performance measurements as defined in TS 28.552 [8].

-	5G End to end KPIs as defined in TS 28.554 [10].

NWDAF shall use the following services to have access to the information provided by OAM:

-	Generic performance assurance and fault supervision management services as defined in TS 28.532 [6].

-	PM (Performance Management) services as defined in TS 28.550 [7].

-	FS (Fault Supervision) services defined in the TS 28.545 [9].

NWDAF can be configured to invoke the existing OAM services to retrieve the management data that are relevant for analytics generation, which may include NF resources usage information (e.g., usage of virtual resources assigned to NF) and NF resource configuration information (e.g., life cycle changes of NF resource configurations).

OAM perform the required configuration in order to provide the information requested by NWDAF subscription and perform the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

Another usage of OAM services is when the target of data collection is related to MDT based retrieval of information which is a per UE mechanism.

# * * * * Second change * * * *

## 6.5	NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "NF load information";

-	The Target of Analytics Reporting: an optional SUPI;

-	Analytics Filter Information: optional S-NSSAI, an optional list of NF Instance IDs, NF Set IDs, or NF types;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If a list of NF Instance IDs (or respectively of NF Set IDs) is provided, the NWDAF shall provide the analytics for each designated NF instance (or respectively for each NF instance belonging to each designated NF Set). In such case the Target of Analytics Reporting should be ignored.

Otherwise, if a SUPI is provided, the NWDAF shall use the SUPI to determine which NF instances (AMF, N3IWF, SMF and PCF) are serving this specific UE, filter them according to the provided S-NSSAI and NF types using data collected from NRF or OAM, and provide analytics for these NF instances.

NOTE:	Only NF instances of type AMF, N3IWF, SMF, and PCF can be determined using a SUPI.

# * * * * Third change * * * *

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1 for the relevant NF instance(s).

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF load | NRF | The load of specific NF instance(s) in their NF profile as defined per TS 29.510 [18]. |
| NF status | NRF | The status of specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g., NF operational or interrupted during virtual/physical resources reconfiguration) as defined in TS 28.533 clause 5.2 [19] |

NOTE 1:	The OAM information can be used as a complement to NRF information for some or all of the following aspects: resources utilization, NRF information correlation, and alternative source of information if NRF information on load is not available.

NOTE 2:	NWDAF can request NRF for data related to NF instances, as described in TS 29.510 [18].

NOTE 3:	NWDAF can correlate the NF resources configuration with NF resource usage for generating the analytics output.

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition to information listed in Table 6.5.2-1.

Table 6.5.2-2: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

# * * * * Fourth change * * * *

### 6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics. NF load statistics information are defined in Table 6.5.3-1. NF load predictions information are defined in Table 6.5.3-2.

Table 6.5.3-1: NF load statistics

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of observed load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) | The maximum load of the NF instance over the Analytics target period |

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of predicted load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) | The maximum load of the NF instance over the Analytics target period |
| > Confidence | Confidence of this prediction |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

# * * * * Fifth change * * * *

### 6.5.4	Procedures

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to NF load information, the target for analytics and the analytics filter are set according to clause 6.5.1. The NF can request statistics or predictions or both and can provide a time window.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF resource usage and NF resources configuration following steps captured in clause 6.2.3.2 for data collection from OAM. Steps 2-5 may be skipped when e.g. the NWDAF already has the requested analytics.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6a.	The NWDAF subscribes to changes on the load and status of NF instances registered in NRF and identified by their NF id from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

6b. NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF along with the corresponding Validity Period, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them along with the corresponding Validity Period to the NF upon reception of notification of new NF load information from OAM.

* * * End of changes * * * *