3GPP TSG-SA2 Meeting #135	S2-1910566

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019	(rev. of S2-1910099)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0003 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Network Performance related network data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Input with incomplete references on RAN - Status of RAN is not provided (ambiguous semantics in analytics) - NF sets are not addressed. - Text not aligned with the conclusions of TR 23.791 (solutions #18 and #28 selected for normative work) - Inconsistencies in Tables 6.6.3-1 and 6.6.3-2, analytics inconsistencies in the case of any UE, UE group, single UE - Constant NF load reporting for every load fluctuation in an area of interest will lead to signalling overhead and resource exhaustion at NWDAF and OAM/NRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Information on AMF RAN status, required for consistent analytics - 3GPP references on RAN and CN load, mobility aspects - Removal of inconsistencies mentionned above - Introduction of Load Reporting Threshold |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.6.2, 6.6.3, 6.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev2 is a merge of rev1 with CR0070. rev3 no longer incorporates the changes from CR0070. |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## 6.6	Network Performance Analytics

### 6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the load, communication and mobility  performance in an Area of Interest; in addition, NWDAF may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifierthat refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested or any UE;

-	Analytics Filter Information containing:

-	Optional maximum number of results;

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise);

-	A Load Reporting Threshold, which indicates the threshold for the NF instance load level in the area of interestwhich when reached or crossed above/below shall be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

Table 6.6.2-1:  Load and Performance information collected by NWDAF

| Load information |  | Source |  | Description |
| --- | --- | --- | --- | --- |
| Status, load and performance information |  | OAM |  | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance per Cell Id in the Area of Interest as defined in TS 28.552 [8]. |
| Load information | NRF |  | Load per NF |  |

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

The NWDAF shall be able to collect UE mobility information as stated in clause 6.7.2.2.

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest |
| > gNB status information | Proportion of gNBs that have been up and running during the entire Analytics target period in the area subset. |
| > gNB resource usage | Average usage of assigned resources (CPU, memory, disk). |
| > Number of UEs | Number of UEs observed in the area subset. |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area |
| > gNB status information | Proportion of gNBs that will be up and running during the entire Analytics target period in the area subset. |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area subset. |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |

NOTE 1: The predictions are provided with a Validity Period, as defined per clause 6.1.3.

NOTE 2: The analytics on number of UEs are related to the information retrieved from the AMF.

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Analytics target period.

### 6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting, Analytics Filter="Area of Interest", "Load Reporting Threshold", and Analytics target Period(s)) to the NWDAF.

2a-2d.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest and subscribes to NF load and status information from NRF about these AMF(s).

3a-3b.	The NWDAF subscribes to OAM services to get the status and load information and the resource usage on the Area of Interest, following the procedure captured in Clause 6.2.3.2.

Editor's note:	The exact SA WG5 service, granularity (NF, eNB) and measurement or KPI that are used for load.

4a-4b.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target of Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

5.	The NWDAF derives the requested analytics.

6.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion).

7-8. A change of network performance information, i.e. change in the load in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics taking into account the most recent data collected.

10.	When relevant according to the Analytics target period or Load Reporting Threshold, the NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion.

* * * End of changes * * * *