3GPP TSG-SA2 Meeting #135	S2-1909305

# Split, Croatia, October 14 – 18, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0074 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retrieval of input data for Analytics ID: “Observed Service Experience” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following corrections are proposed: - The input data to determine Observed Service Experience is collected from several NF using different Event ID, i.e. Change of UE location for AMF and a new Event Id for SMF. The procedure does not indicate which Event Id should be used. - The Application ID is available at the SMF, therefore retrieving this data from PCF does not add any value and increases the complexity in the procedure. - The Event Id is related to an Application Id, so there is no Service Experience across multple Application Ids, but mutltiple requests from the NF consumer, one per Application Id (or alternatively multipel Application Ids should be included in the request and response for analytics) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure is updated, a new Event Id is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2, 6.4.3, 6.4.4, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 1824 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	global Service Experience (i.e. for all UEs) for a Network Slice, given an Application or a set of Applications in a Network Slice;

-	detailed Service Experience (i.e. a group of UEs or a list of UEs) for a given Application.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information: one or more combinations of the following Analytics Filters: Area of Interest, Media/application bandwidth, S-NSSAI, DNN, Application ID;

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provides statistics and predictions on the observed service experience to a consumer NF or to OAM.

NOTE:	OAM uses the Observed Service Experience as defined in SA WG5 TSs.

### 6.4.2	Input Data

The service data collected from the AF and the network data from other 5GC NFs for observed service experience are defined in Table 6.4.2-1 and Table 6.4.2-2.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | the time when the information is collected. |
| Location Info | AMF | The UE location information. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for which QoE metrics are requested.w |
| IP filter information | SMF | Used by NWDAF to identify the service data flow template for the application for which QoE metrics are requested. |
| QFI | SMF | QoS Flow Identifier(s) assigned for the Application traffic |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission in a given time window |
| Packet retransmission | UPF | The observed number of packet retransmission in a given time window. |

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

The NWDAF requests the QFI assigned to an Application Identifier to the SMF, the request can be for a SUPI, for an Internal-Group-Id or for any UE. The Observed QoS measurements per QFI are obtained from UPF, how the NWDAF obtains this is not specified in this Release.

The NWDAF requests the UE location to the AMF, the request can be for a SUPI, for an Internal-Group-Id or for any UE.  The NWDAF correlates the input data provided by AMF and SMF, using the parameters in Table 6.2.2.1-2.

When input data is requested for a SUPI, the NWDAF finds the AMF(s) serving the SUPI and the SMF(s) serving the SUPI PDU sessions as defined in Table 6.2.2.1-2

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI (optional) | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) | List of observed service experience information for each Application. |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3. |
| Slice service experience (optional) | Service experience for an Application on a Network Slice over the Analytics target period (average, variance). |

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI (optional) | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) | List of predicted service experience information for each Application. |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area, where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Areas of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3. |
| > Confidence information | Confidence of this prediction. |
| Slice service experience (optional) | Service experience for an Application on a Network Slice over the Analytics target period (average, variance). |

The number of Service Experiences and SUPIs is limited by the maximum number of results provided as input parameter.

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for subscription to Analytics Id on Service Experience for an Application Id

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = Any UE) as defined TS 23.502 [3].

NOTE 1:	In the case of external AF, NEF translates the requested Area of Interest into a list of geographic zone identifier(s) as described in clause 5.6.7.1 of TS 23.501 [2].

2b.	NWDAF subscribes to Event Id “Changes of UE Location” from the AMF using Namf_EventExposure Subscribe including an indication that any UE is targeted, the NWDAF sends the request to multiple AMFs.

2c. NWDAF subscribes to Event Id “QFI allocation” from the SMF using Nsmf_EventExposure Subscribe including anindication that any UE is targeted, the NWDAF sends the request to multiple SMFs.

2c., The NWDAF estimates the Service experience for the application using the input data listed in clause 6.4.2.

NOTE 2:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

Editor's notes:	The explicit list of input data from OAM (such as PM measurements) is not finalised and FFS.

NOTE 3:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify as defined in caluse 6.4.3.,

NOTE 4:	The call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

If the consumer NF is PCF, and for an Application Id, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies), then if NWDAF reports to PCF an Observed Service Experience that indicates  that the application SLA is not satisfied, the PCF may determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 5:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

### 6.4.5	Procedures to request Service Experience reported for UEs on a Network Slice

This procedure is similar to the procedure in clause 6.4.4.

The difference compared to the procedure in clause 6.4.4 is that it allows the consumer to request Analytics ID "Service Experience" for a list of SUPI(s) or Internal Group Identifier(s). The NWDAF requests input data for a SUPI or for an Internal-Group-Id from the AMF or from the SMF.

Therefore, Observed Service experience may be provided individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application on a Network Slice.

.

* * * * End of Change * * * *