3GPP TSG-SA WG2 Meeting #135	S2-1909588

# Split, Croatia, 14 – 18 October 2019	(revision of S2-19xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1825 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IWK IDLE mode mobility using N26 interface with I-SMF/I-UPF and data forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When UE is in CM-IDLE mode with data buffering due to power saving state in EPS/5GS and move to 5GS/EPS with N26 interface, there is no I-SMF/I-UPF for related forwarding data handling in mobility procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Introduce Idle mode mobility procedures from 5GS to EPS with data forwarding and I-SMF removal. 2) Introduce Idle mode mobility procedures from EPS to 5GS with data forwarding and I-SMF inserstion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing I-SMF handling in idle mode interworking procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.2a (new), 4.23.12.3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The procedures below refer to procedures introduced by other papers. The reference to 4.11.1.3.3A in clause 4.23.12.3a is covered by 23.502 CR1590 rev 2 (S2-1909341). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

#### 4.23.12.2a.	5GS to EPS Idle mode mobility using N26 interface with Data forwarding and I-SMF removal

The considered scenario is that a UE has PDU sessions with I-SMF, I-UPF available and data buffered in 5GS. The I-SMF is removed after idle mode mobility to EPS and data forwarding from I-UPF to SGW.

Figure 4.23.12.2a-1: 5GS to EPS Idle mode mobility using N26 interface with data forwarding and I-SMF removal

1.	Step 1-4 from clause 4.11.1.3.2.

2.	Step 5 from clause 4.11.1.3.2 with the following differences:

-	PGW-C+SMF is replaced by I-SMF.

-	PGW-U+UPF is replaced by I-UPF.

-	The SMF includes the Buffered DL Data Waitting indication in the Nsmf_PDUSessionResponse, if the Extended buffering timer is running in SMF.

3.	Step 6 from clause 4.11.1.3.2 with the following enhancement:

The AMF includes the Buffered DL Data Waitting indication received from SMF in step 2.

4 - 14. 	Steps 7-18 from clause 4.11.1.3.2 with following enhancement:

MME delays the sending of Contxt Acknowledge to AMF until step 22 below.

15-18.	If the "Buffered DL Data Waiting indication" is set in step 3, the MME triggers steps 11-14 in clause 5.3.3.1A of TS 23.401.

19-21. Steps 10a, b, c from clause 4.11.1.2.1 with the following differences:

-	PGW-C+SMF is replaced by I-SMF. PGW-U+UPF is replaced by I-UPF.

-	The I-SMF and I-UPF does not allocate any CN forwarding tunnel info to NG-RAN.

-	The I-SMF and I-UPF forwards data using the provided SGW forwarding tunnel in step 19. The I-SMF also initiate a timer to release the SM context of the PDU session.

The I-UPF forwards the buffered DL data to the Serving GW. The data received by the Serving GW on the forwarding tunnel is forwarded on the (newly) established tunnel to eNB.

22.	The I-SMF initiates SM context release after the timer in step 21 is expired.

***** Next Change *****

##### 4.23.12.3a. 	EPS to 5GS Idle mode mobility using N26 interface with Data forwarding and I-SMF insertion

The considered scenario is that a UE has PDN connections in EPS, during idle mode mobility from EPS to 5GS, I-SMF need to be inserted and data forwarding from SGW to I-UPF is performed.

Figure 4.23.12.3a-1: EPS to 5GS Idle mode mobility using N26 interface with data forwarding and I-SMF insertion

1. Steps 1-10 of clause 4.11.1.3.3A are performed.

2.	The AMF determines whether a I-SMF needs to be selected based on UE location and service area of the PGW-C+SMF, if new I-SMF needs to be selected, the AMF selects a new I-SMF as described in clause 4.23.2.

Case: I-SMF insertion.

3.	Step 4 of clause 4.11.1.3.3A is performed with following differences:

-	PGW-C+SMF is replaced by I-SMF.

4.	The new I-SMF selects a new I-UPF: Based on the received information from AMF, e.g. EPS PDN connections, UE location information, the new I-SMF selects a new I-UPF as described in clause 6.3.3 of TS 23.501 [2].

5.	The new I-SMF initiates a N4 Session Establishment to the new I-UPF. In case tunnel endpoint is allocated by the new I-UPF, the new I-UPF provide tunnel endpoints to the new I-SMF, otherwise the new I-SMF allocates the tunnel endpoints and provides them to the new I-UPF.

6a.	The new I-SMF invokes Nsmf_PDUSession_Create Request (new I-UPF DL tunnel information, EPS PDN Connection context, UE location Information) towards the PGW-C+SMF. The I-SMF selects the PGW-C+SMF using the PGW-C+SMF info received from AMF. The PGW-C+SMF finds the corresponding PDU Session based on the PDN Connection Context in the request.

The SM Context ID at I-SMF is to be used by the SMF for further PDU Session operation, e.g. to notify the new I-SMF of PDU session release.

6b.	The PGW-C+SMF initiates N4 Session Modification toward the PDU Session Anchor UPF, providing the new I-UPF DL tunnel information. The PSA begins to send the DL data to the new I-UPF as indicated in the new I-UPF DL tunnel information.

6c.	The PGW-C+SMF responds to the new I-SMF with Nsmf_PDUSession_Create Response including the tunnel info for PDU Session.

7.	The I-SMF initiates N4 Session Modificaiton toward the new I-UPF including the tunnel info provided from SMF.

8.	The new I-SMF sends a Nsmf_PDUSession_CreateSMContext Response (N2 SM information (PDU Session ID, QFI(s), QoS profile(s), S-NSSAI, User Plane Security Enforcement, UE Integrity Protection Maximum Data Rate), Cause)) to the AMF. The N2 SM information, including the UL Tunnel infor for of the new I-UPF, formulated by I-SMF based on the Buffered DL data waiting indciaiton in step 3 above.

If the PDU Session has been assigned any EPS bearer ID, the new I-SMF also includes the mapping between EPS bearer ID(s) and QFI(s) into the N2 SM information to be sent to the NG-RAN.

9. 	Step 12-16 of clause 4.11.1.3.3A are performed.

Case: I-SMF insertion.

10.	Step 17a-17c of clause 4.11.1.3.3A are performed with following differences:

-	PGW_C+SMF and UPF are replaced by I-SMF and I-UPF.

-	The I-SMF and I-UPF shall also allocate the data forwarding tunnel for EPS if Buffered DL Data Waitting Indicaiton is received in step 3 above.

The Serving GW forwards the buffered DL data to the I-UPF. The data received by the I-UPF on the forwarding tunnel is forwarded on the (newly) established N3 tunnel to the NG-RAN.

11. 	Step 18-20 of the clause 4.11.1.3.3A are performed.

***** End of Changes *****