SA WG2 Meeting #135	S2-1909991

# 14 - 18 Oct 2019, Split, HR														 (revision of S2-1909299)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0024 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to delivery of PC5 QoS parameters to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PC5 QoS parameters to NG-RAN is provided from PCF to the AMF, the AMF stores such information as part of the UE context and provides those parameters to NG-RAN using NGAP. In the current solution, UE Configuration Update procedure is use for the purpose; however, UE Configuration Update procedure is to update UE configuration, in which the UE policy is encapsulated in a container to be delivered to the UE. Additionally, in UE Configuration Update procedure, PCF provides the PC5 QoS parameters to NG-RAN in the Namf_Communication_N1N2MessageTransfer message, which implies that the information is transparent to AMF. However, AMF needs to store it as the UE context. There’s no existing 23.502 procedures that can be reused directly for delivering PC5 QoS parameters to NG-RAN, thus it is proposed to document that in a new procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies how the UE policy control association procedure is used for delivering PC5 QoS parameters used by NG-RAN to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong procedure used for delivering PC5 QoS parameters to NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.5.7	Delivery of PC5 QoS parameters to NG-RAN

The Npcf_UEPolicyControl_Create service operation for UE is used to provide PC5 QoS parameters to NG-RAN and performed during UE registration procedure as defined in TS 23.502 [7] with the following additions:

-	When receiving Npcf_UEPolicyControl_Create Request from the AMF which includes the PC5 capability for V2X, PCF generates the PC5 QoS parameters for the UE as defined in clause 5.4.2 used by NG-RAN.

-	In the roaming case, the H-PCF includes the PC5 QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message sent to V-PCF in a NG-RAN PC5 policy container, and V-PCF relays this NG-RAN PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the PC5 QoS parameters in a N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF determines that the N2 container including the PC5 QoS parameters comes from (V-)PCF, the AMF stores such information as part of the UE context.

The AMF includes the PC5 QoS parameters in the NGAP message to the NG-RAN.

END OF CHANGES