3GPP TSG SA2 Meeting #135	S2-1910249

# Split, Croatia 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0010 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting wireline non-3GPP access in the AM Policy Association |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 04-10-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The AMF determines the Access Type and RAT type based on the Global RAN Node ID associated with the N2 interface, as described in clause 7.2.1.1 step 12. may indicate to the PCF that the 5G-RG or FN-RG registered via a wireline non 3GPP access, using both the Access Type and the RAT type. Then, RAT type reporting for wireline should be possible for FN-RG and handover for 5G-RG since AMF can use the N2 Global RAN Node ID type set to W-AGF. There is mechanisim to different BBF and CableLabs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the input information provided by the AMF with the Access Technology Type set to “wireline”. # Clarify that the FN-RG Registration procedure may: - Include validations of the Service Area Restritions for a FN-CRG, as defined in clause 4.3.3.5. - AMF determination and reporting of both the Access type and RAT type to PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of the RAT type reporting to PCF for non 3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 8.2	Npcf_AMPolicyControl service

### 8.2.1	Npcf_AMPolicyControl_Create service operation

The input data listed in clause 5.2.5.2.2 in TS 23.502 [3] applies when an AM Policy Association is created for a 5G-RG, except for the handling of RFSP information.

The input information when the UE registers via W-5GAN includes the Access type set to non-3GPP access, the User Location Information including the Line ID or the HFC node Id.

The output information when the UE registers via W-5GAN is defined in clause 9.5 and the Policy Control Request triggers applicable for non-3GPP access type, when UE accesses via W-5GAN are defined in Table 8.2.1-1.

Table 8.2.1-1: Policy Control Request Triggers relevant for AMF and non-3GPP access type wireline access

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Service Area restriction change | The subscribed service area restriction information has changed. | PCF (AM Policy) |
| Change of the Allowed NSSAI | The Allowed NSSAI has changed | PCF (AM Policy) |
| Access Type change | The Access Type and the RAT Type has changed | PCF (AM Policy) |
| NOTE: The RAT type is reported for 3GPP access.or when the 5G-RG or FN-RG registers over wireline access (i.e. W-AGF). |  |  |

### 8.2.2	Npcf_AMPolicyControl_Update service operation

The input data listed in clause 5.2.5.2.5 in TS 23.502 [3] applies when an AM Policy Association is updated for a 5G-RG or for a FN-CRG., except for the notification of UE location, PRA changes or RFSP index change.

The Access type change trigger requests the AMF to report a new Access Type and RAT Type when the UE simultaneous connects over both 3GPP and wireline non-3GPP access type.  The AMF reports the list of Access Type and RAT combinations available in the UE access and mobility context in the Npcf_AMPolicyControl_Update service operation.

The output information when the UE registers via W-5GAN is defined in clause 9.5 and the Policy Control Request triggers applicable for non-3GPP access type, when UE accesses via W-5GAN are defined in Table 8.2.1-1.

End of changes