SA WG2 Meeting #136AH	S2-2000436

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-2000xxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2010 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deregistration procedure for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-12-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no procedure for Deregistration procedure for ATSSS. Per TS 23.501, - When the UE deregisters from one access (but remains registered on the other access), the AMF informs the SMF that an access type becomes unavailable for the MA PDU Session. Subsequently, the SMF notifies the UPF that the access type has become unavailable. Therefore, if the deregistration procedure applies to the single access, the MA PDU session should be still available in the other access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE or network requested Deregistration procedures procedure for ATSSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network cannot perform Deregistration procedures for MA PDU Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.9.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.9.X	Deregistration procedures

4.22.9. X.1	General

Deregistration procedure is used to deregister the UE from a single access or both accesses when the UE has any established any MA PDU Session(s). If the UE is deregistered from both 3GPP and non-3GPP accesses, all the MA PDU session(s) are released over both accesses. If the UE is deregistered from a single access, all the MA PDU session(s) are released over single accecss.

4.22.9.X.2	 UE-initiated Deregistration

The UE initiates the Deregsitration procedure using the procedure as described in Figure 4.2.2.3.2-1, with the following differences and clarifications:

-	In step 2, AMF to SMF: Nsmf_PDUSession_ReleaseSMContext (SUPI, PDU session ID, access type). For each MA PDU session, the AMF includes the received access type in step 1.

-   In step 3, for each MA PDU session, the SMF releases the resources over the  access(es) based on the access type received from the AMF.

-   In step 3a, The SMF sends N4 Session Release Request (N4 Session ID, access type) message to the UPF(s) of the PDU Session. The UPF(s) shall drop any remaining packets of the PDU Session over the single access type and release all tunnel resource and contexts associated with the N4 Session over the access(es) based on the received access type.

-   In step 4, if the UE is deregistered the SMF updates the access type for the MA PDU session to the access which the UE remains registered on.

-   In step 7, for each MA PDU session, the UE updates the access type for the MA PDU session to the access which the UE remains registered on.

4.22.9.X.3	 Network-initiated Deregistration

The Network initiates the Deregistration procedure using the procedure as described in Figure 4.2.2.3.3-1, with the following differences and clarifications:

- In step 4, if the UE has any established MA PDU Session over the target access for deregistration indicated in step 2, then step 2 ~ step 5 of UE-initiated Deregistration procedure in clause 4.22.9.X.2 is performed.

- In step 6, if the UE receives the Deregistration Request message over single access type from the AMF in step 2, the UE updates the access type for the MA PDU session to the access which the UE remains registered on.

* * * * End of Changes * * * *