3GPP TSG-SA WG2 Meeting #136AH 	S2-2000803

Jan 13 – 17, 2020, Incheon, Korea	 (revision of S2-200xxxx)

Source:	CATT

Title:	Discussion on local LMF in NG-RAN

Document for:	Discussion/ Approval

Agenda Item:	7.6

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: this discussion paper aims at discussing the benefit of local LMF in NG-RAN.

1. Introduction

As indicated in LS RP-193262 [1], RAN discussed LMC, but has not reached consensus on any follow-on work. Questions were raised during discussion about the benefit and deployment scenario.

This discussion paper analyses the two architectures from latency and mobility aspects and proposes the benefit and deployment scenario for LMF in NG-RAN architecture.

2. Discussion

## 2.1 Latency analysis

This clause is proposed to analyse latency in two architectures, i.e. LMF in CN and LMF in NG-RAN. The difference of the two architectures is different LMF location. Because LMF location does not impact positioning method (e.g. measurement, positioning computation), so latency introduced by positioning method is not considered.

Based on procedures in TS 23.273, the steps and procedures impacted by LMF location are as follows:

-	5GC-MT-LR and 5GC-MO-LR procedures (the following steps are same):

-	Steps 6-9 in 5GC-MT-LR procedure for the regulatory location service in clause 6.1.1.

-	Steps 10-13 in 5GC-MT-LR procedure for the commercial location service in clause 6.1.2.

-	Steps 3-6 in 5GC-MO-LR procedure in clause 6.2.

-	Initiation and Reporting of Location Events in Deferred 5GC-MT-LR procedure in clause 6.3.1: Steps 13-18 and steps 25-27.

-	Event Reporting with no change of LMF procedure in Low Power Periodic and Triggered 5GC-MT-LR procedure in clause 6.7.1: Steps 3-8a.

So only the latency of the steps above is analysed.

2.1.1	5GC-MT-LR and 5GC-MO-LR procedures

The steps in 5GC-MT-LR and 5GC-MO-LR procedure impacted by different architecture (i.e. LMF in CN and LMF in NG-RAN) are as follows:

-	Steps 6-9 in 5GC-MT-LR procedure for the regulatory location service in clause 6.1.1.

-	Steps 10-13 in 5GC-MT-LR procedure for the commercial location service in clause 6.1.2.

-	Steps 3-6 in 5GC-MO-LR procedure in clause 6.2.

The details of the steps above in different architecture are shown in Figure 1.

Figure 1: Steps in 5GC-MT-LR and 5GC-MO-LR procedures in different architecture

Based on the positioning method (UE assisted or Network assisted), latency of the steps above are different.

When UE Assisted and UE Based Positioning procedure is used, latency analysis is shown in Table 1.

Table 1: Latency analysis for steps in Figure 1 when UE assisted and UE based positioning procedure is performed

| Operations and latency | Latency in different architecture |  |
| --- | --- | --- |
|  | LMF in CN | LMF in NG-RAN |
| LMF selection | 1ms | 0 |
| Delivery between AMF and LMF (5 ms) | 20ms (including steps 2, A-1, A-7, 4) | 0 |
| Delivery on N2 interface (10ms) | 20ms (including steps A-3, A-6) | 20ms (including steps 2, 4) |
| Delivery on radio interface (2ms) | 4ms (including steps A-3, A-6) | 4ms (including steps A-1, A-4) |
| Total latency | 45ms | 24ms |

Note: the delivery latency is referenced from R2-1901279[2]

When Network Assisted Positioning procedure is used, latency analysis is shown in Table 2.

Table 2: Latency analysis for steps in Figure 1 when network assisted positioning procedure is performed

| Operations and latency | Latency in different architecture |  |
| --- | --- | --- |
|  | LMF in CN | LMF in NG-RAN |
| LMF selection | 1ms | 0 |
| Delivery between AMF and LMF (5ms) | 20ms (including steps 2, B-1, B-6, 4) | 0 |
| Delivery on N2 interface (10ms) | 20ms (including steps B-3, B-5) | 20ms (including steps 2, 4) |
| Total Latency | 41ms | 20ms |

According to Figure 1, Table 1 and Table 2, the LMF in NG-RAN architecture could save delivery time between AMF and LMF and LMF selection latency, the total reduction is 21ms.

2.1.2	Initiation and Reporting of Location Events in Deferred 5GC-MT-LR procedure

The steps in Initiation and Reporting of Location Events in Deferred 5GC-MT-LR procedure impacted by different architecture (i.e. LMF in CN and LMF in NG-RAN) are as follows:

-	Steps 13-18 and steps 25-27 in Initiation and Reporting of Location Events in Deferred 5GC-MT-LR procedure in clause 6.3.1.

The details of the steps above in different architecture are shown in Figure 2.

Figure 2: Steps in deferred 5GC-MT-LR procedure in different architecture

Based on the positioning method (UE assisted or Network assisted), latency of the steps above are different.

When UE Assisted and UE Based Positioning procedure is used (i.e. step 27 is the step A in Figure 1), latency analysis is shown in Table 3.

Table 3: Latency analysis for steps in Figure 2 when UE assisted and UE based positioning procedure is performed

| Operations and latency | Latency in different architecture |  |
| --- | --- | --- |
|  | LMF in CN | LMF in NG-RAN |
| Delivery between AMF and LMF (5ms) | 30ms (including steps 16a, 17b, 25b, 26a, A-1 and A-7 in Figure 1) | 0 |
| Delivery on N2 interface (10ms) | 60ms (including steps 16b, 17a, 25a, 26b, A-3 and A-6 in Figure 1) | 0 |
| Delivery on radio interface (2ms) | 12ms (including steps 16b, 17a, 25a, 26b, A-3 and A-6 in Figure 1) | 12ms (including steps 16, 17, 25, 26, A-1 and A-4 in Figure 1) |
| Total Latency | 102ms | 12ms |

When Network Assisted Positioning procedure is used, latency analysis is shown in Table 4.

Table 4: Latency analysis for steps in Figure 2 when network assisted positioning procedure is performed

| Operations and latency | Latency in different architecture |  |
| --- | --- | --- |
|  | LMF in CN | LMF in NG-RAN |
| Delivery between AMF and LMF (5ms) | 30ms (including steps 16a, 17b, 25b, 26a, B-1 and B-6 in Figure 1) | 0 |
| Delivery on N2 interface (10ms) | 60ms (including steps 16b, 17a, 25a, 26b, B-3 and B-6 in Figure 1) | 0 |
| Delivery on radio interface (2ms) | 8ms (including steps 16b, 17a, 25a, 26b) | 8ms (including steps 16, 17, 25, 26) |
| Total latency | 98ms | 8ms |

According to Figure 2, Table 3 and Table 4, the LMF in NG-RAN architecture could save delivery time between AMF and LMF and delivery time on N2 interface, the total reduction is 90ms.

2.1.3	Event Reporting with no change of LMF procedure in Low Power Periodic and Triggered 5GC-MT-LR procedure

The steps in Low Power Periodic and Triggered 5GC-MT-LR procedure impacted by different architecture (i.e. LMF in CN and LMF in NG-RAN) are as follows:

-	Steps 3-8a in Event Reporting with no change of LMF procedure in clause 6.7.1.

The details of the steps above in different architecture are shown in Figure 3.

Figure 3: Steps in low power deferred 5GC-MT-LR procedure in different architecture

Latency analysis of the steps shown in Figure 3 is shown in Table 5.

Table 5: Latency analysis for steps in Figure 3

| Operations and latency | Latency in different architecture |  |
| --- | --- | --- |
|  | LMF in CN | LMF in NG-RAN |
| Delivery between AMF and LMF (5ms) | 10ms (including steps 5, 6) | 0 |
| Delivery on N2 interface (10ms) | 20ms (including steps 4, 7) | 0 |
| Delivery on radio interface (2ms) | 4ms (including steps 3, 8) | 4ms (including steps 3, 8) |
| Total Latency | 34ms | 4ms |

According to Figure 3 and Table 5, the LMF in NG-RAN architecture could save delivery time between AMF and LMF and delivery time on N2 interface, the total reduction is 30ms.

2.1.4	Summary

Based on analysis above, the LMF in NG-RAN architecture can reduce the positioning procedure latency. The summery is shown in Table 6.

Table 6: Latency reduction summery

| Procedures | Positioning method | Latency in different architecture |  | Latency reduction in the architecture of LMF in NG-RAN |
| --- | --- | --- | --- | --- |
|  |  | LMF in CN | LMF in NG-RAN |  |
| 5GC-MT-LR and 5GC-MO-LR procedures | UE Assisted and UE Based Positioning procedure | 45ms | 24ms | 21ms |
|  | Network Assisted Positioning procedure | 41ms | 20ms | 21ms |
| Deferred 5GC-MT-LR procedure | UE Assisted and UE Based Positioning procedure | 102ms | 12ms | 90ms |
|  | Network Assisted Positioning procedure | 98ms | 8ms | 90ms |
| Low Power Periodic and Triggered 5GC-MT-LR procedure |  | 34ms | 4ms | 30ms |

Proposal 1: The benefit of LMF in NG-RAN architecture is reducing the positioning procedure latency which is important to support use cases requesting low positioning latency.

## 2.2 Mobility analysis

The following table is extracted from clause 8.1.7 in TR 22.804. Based on the table, IIoT scenario requires low positioning latency. For some use cases, the positioning latency is 10ms.

Because LMF in NG-RAN architecture can reduce latency by 90ms in periodic or triggered Deferred 5GC-MT-LR procedure which can be used by IIoT device to report location, the architecture is meaningful for IIoT scenario.

| Scenario | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility | Use case reference |
| --- | --- | --- | --- | --- | --- | --- |
| Augmented reality in smart factories | < 1 m | 99% | < 0,17 rad | < 15 ms | < 10 km/h | Factories of the future 10.8 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | 99.9% | N/A | 10 ms | < 30 km/h | Factories of the Future15.5 |

Furthermore, there are low mobility UEs in the IIoT scenario, it is not needed to consider LMF change.

Proposal 2: LMF in NG-RAN architecture can support IIoT scenario which includes low mobility UEs and requests low positioning latency.

3. Proposal

Proposal 1: The benefit of LMF in NG-RAN architecture is reducing the positioning procedure latency which is important to support use cases requesting low positioning latency.

Proposal 2: LMF in NG-RAN architecture can support IIoT scenario which includes low mobility UEs and requests low positioning latency.

4. References

[1] RP-193263, LS on Local NR positioning in NG-RAN, TSG RAN#86, Spain, 9-12 Dec 2019

[2] R2-1901279, Considerations on RAN based positioning architecture, RAN2#105, Greece, 25 Feb -1Mar 2019