SA WG2 Meeting #S2-136AH	S2-2000554

13 - 17 January, 2020, Incheon, South Korea	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2070 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Traffic Forwarding issue at UPF side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification is unclear how to handle multiple NW-TT ports per UPF. It is not possible to know a priori the NW-TT port that needs to be bound to a PDU session, since traffic forwarding information is calculated and configured on a later phase, i.e., configuration phase. Given the agreement that the 5GS models a TSN bridge, the UPF needs to be able to bridge Ethernet traffic between the device side ports and the network side ports in the general case. Just as there is no binding between PDU sessions and network interfaces for IP traffic, and traffic is routed to the appropriate outgoing interface using a routing table, for Ethernet traffic there should not be any binding between network ports, and frames should be bridged based on traffic forwarding information. A single device may need to communicate via any of the network side ports depending on which host it communicates with. However, the specification text still establishes a binding between DS-TT port and NW-TT port. Therefore, we propose to change the text in order to remove this binding. In Rel. 16, we propose to adopt general community recommendation that the TSN system configures the forwarding rules statically. In simple networks the 5GS may be able to configure the forwarding rules locally. This information is general for any bridge, and not only for TSN (not from IEEE 802.1Qbv). The forwarding traffic information is related to the path of every TSN Stream to the TSN AF, but this information is not linked to a TSN Stream ID itself, but to the destination MAC address, VLAN ID, and the egress port of the 5G Bridge. Forwarding rules are independent on the ingress port, so information on the ingress port is not necessary. Note that information based on 802.1Q clauses 8.8.1/8.8.2 is limited as it is only one of the options to do traffic forwarding. It also implies significant updates at every port whenever the information is changed. We propose to leave it more general and allow traffic forwarding rules/information to be on a per bridge basis (instead of per port basis as in 802.1Q, 8.8.1/8.8.2). We also clarify the text in the last paragraph of clause 5.28.2 adding “using AF influence on traffic routing”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove text that states any binding relationship between DS-TT and NW-TT ports. Specify that the UPF forwards traffic based on traffic forwarding information. Specify that the traffic forwarding rules/information are configured statically by the TSN system, and update the list of parameters related to traffic forwarding information. Remove references to 802.1Q, clauses 8.8.1 and 8.8.2. Remove traffic forwarding information that is applied to the ports in the PMI container. Add this text: “using AF influence on traffic routing” to the last paragraph of clause 5.28.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limitation of connectivity in the TSN system. Incorrect binding between DS-TT port and NW-TT port. Incomplete traffic forwarding information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1, 5.28.2, 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side and a PDU Session during reporting of logical TSN bridge information. The TSN AF also stores the information about ports on the UPF/NW-TT side. The UPF forwards traffic to the appropriate egress port based on the traffic forwarding information received from the TSN system.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5G virtual bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge Address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	Bridge Name;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97]:

- Chassis ID subtype and Chassis ID of the 5GS Bridge

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Bridge ID of the 5GS Bridge, port numbers of the Ethernet in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment and stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT of the related PDU session, and port numbers and MAC addresses of the Ethernet ports in NW-TT to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

* * * * Next Change * * * *

### 5.28.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The QoS parameters mapping for TSN is described in TS 23.503 [45] clause 6.1.3.23.

The configuration information of 5GS Bridge as defined in 802.1Qbv [96] and in 802.1Q [98], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities;

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

The configuration information of 5GS Bridge as defined in 802.1Q [98], includes the following:

-	Chassis ID of 5GS Bridge:

-	Traffic forwarding information is configured statically by the TSN system:

-	Destination MAC address and VLAN ID of TSN stream;

-	The egress port of the 5GS Bridge.

-	Configuration information per stream according to 802.1Q [98] clause 8.6.5.1:

-	Ingress port number of 5GS Bridge, i.e., ports on DS-TT/NW-TT;

-	Stream priority;

The SMF report the MAC address of the DS-TT port of the related PDU Session to AF via PCF as the MAC address of the PDU Session. The association between the MAC address used by the PDU Session, 5GS Bridge ID and port number on DS-TT is maintained at AF and further used to assist to bind the TSN traffic with the UE’s PDU session.

In the case of provisioning traffic routing information using AF influence on traffic routing based the information above, the AF determines the MAC address used by the PDU Session for the TSN traffic based on ingress port number or egress port number, and requests to PCF influence traffic routing for TSN traffic as defined in clause 5.6.7 with the MAC address of the PDU Session, traffic class, destination MAC address and VLAN ID.

* * * * Next Change * * * *

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. Table 5.28.3.1-1 lists standardized port management information.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see Note 6) |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |
| General |  |  |  |  |
| Port management capabilities (see Note 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminBaseTime | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlList | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminCycleTime (see Note 3) | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| AdminControlListLength (see Note 3) | X | X | RW | IEEE 802.1Qbv [96] Table 12-28 |
| Tick granularity | X | X | R | IEEE 802.1Qbv [96] Table 12-28 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | (x) | X | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| TTL | (x) | X | R | IEEE 802.1AB[97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (see Note 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Per-Stream Filtering and Policing information |  |  |  |  |
| StreamHandleSpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PrioritySpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| StreamGateInstanceID | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PSFPAdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT and NW-TT performs neighbor discovery on behalf on DS-TT. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, which performs neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; (x) = applicable when validation and generation of LLDP frames is processed at the DS-TT; (X) = applicable when validation and generation of LLDP frames is processed centrally at NW-TT. |  |  |  |  |

Exchange of port management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port;

2)	send port management information for a DS-TT or NW-TT Ethernet port;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container whether it wants to retrieve or send port management information or intends to (un-)subscribe for notifications.

* * * * End of Change * * * *