3GPP TSG-WG SA2 Meeting #136AH 	S2-2000724

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2051 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on implementation of Ninef_EventExposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT3 LS S2-2000011/C3-194445 indicated that stage 3 implement the functionality of I-NEF Event Exposure by Nnef_EventExposure service. TS 23.502 should clarify about the implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that Ninef_EventExposure service can be merged into Nnef_EventExposure service with a NOTE. Change the Target of Event Reporting of Ninef_EventExposure service from external UE ID to internal UE ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.3a; 5.2.6A.1; 5.2.6A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2.3a	I-NEF service operations information flow

The procedure is used by the NF (AMF, SMF) in VPLMN to provision monitoring configurations and to explicitly cancel a previous monitoring configuration at I-NEF. Cancelling is done by sending Ninef_EventExposure_UnSubscribe request identifying the monitoring configuration to cancel. The notification steps 3 to 5 are not applicable in cancellation case.

Figure 4.15.3.2.1-1: Ninef_EventExposure_Subscribe, Unsubscribe and Notify operations

1.	NEF or UDM subscribe or unsubscribe to the notification of monitoring events at SMF or AMF as refered to steps 1 to 2 in clause 4.15.3.2.1, steps 1 to 3 in clause 4.15.3.2.2, and steps 1 to 5 in clause 4.15.3.2.3.

2.	AMF or SMF determines that the UE is in roaming, it requests to provision or delete monitoring configuration on I-NEF in Ninef_EventExposure_Subscribe or Ninef_EventExposure_UnSubscribe request. The monitoring configuration on I-NEF includes the notification endpoint information of the NEF. I-NEF acknowledges the execution of Ninef_EventExposure_Subscribe or Ninef_EventExposure_UnSubscribe request. When AMF provisions the monitoring configuration on I-NEF, the I-NEF updates the monitoring event subscription on AMF with the notification endpoint information of the I-NEF by means of Namf_EventExposure_Subscribe message.

NOTE:	In Stage 3 the Ninef_EventExposure service is implemented by the Nnef_EventExposure service.

3.	[Conditional- depending on the Event] refer to step 6e in clause 4.15.3.2.3.

4.	The AMF detects the related event occurs, it sends the event report by means of Namf_EventExposure_Notify message to the I-NEF.

5.	The I-NEF performs normalization of reports and generation of charging/accounting information as defined in clause 6.2.5a of TS 23.501 [2]. The I-NEF sends the event report by means of Ninef_EventExposure_Notify message to the NEF.

# * * * * Second change * * * *

#### 5.2.6A.1	General

The following table shows the I-NEF Services and Service Operations:

Table 5.2.6A.1-1: NF Services provided by the I-NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ninef_EventExposure | Subscribe | Subscribe/Notify | AMF, SMF |
|  | Unsubscribe |  | AMF, SMF |
|  | Notify |  | NEF, UDM |

NOTE:	In Stage 3 the Ninef_EventExposure service is implemented by the Nnef_EventExposure service.

# * * * * Third change * * * *

##### 5.2.6A.2.2	Ninef_EventExposure_Subscribe operation

Service operation name: Ninef_EventExposure_Subscribe

Description: the consumer subscribes to an event with monitoring configuration in I-NEF.

Inputs (required): NF ID, (Set of) Event ID(s) as specified in clause 4.15.3.1, Target of Event Reporting (UE(s) ID (SUPI or Internal Group Identifier or indication that any UE is targeted)), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), Subscription Correlation ID.

Inputs (optional): Event Filter, Expiry time, Chargeable Party Identifier.

Outputs (required): Expiry time (required if the subscription can be expired based on the operator's policy), cause.

Outputs (optional): None.

# * * * * End of changes * * * *