3GPP TSG-SA2 Meeting #136-AH	S2-2000223

# Incheon, Korea (Republic Of), 13th Jan 2020 - 17th Jan 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2024 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scope parameter for binding indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has already been agreed that an NF service consumer may subscribe via another network function for notifications. And that two binding indications are then required. For example, NF_A may subscribe to NF_B on behalf of NF_C. NF_A additionally subscribe to subscription related events. Further, the possibility to provide NF service instance as part of the binding has been removed at the previous meeting because it was unclear to which service the instance ID would relate to. The possibility to allow for a service level binding seems desirable to comply with the overall principles of binding. At the previous meeting, the possibility to indicate the service a binding relates to was already introduced. But this is restricted to notifications. As the binding for notifications and the binding for other services could be described in the same request message, and even binding for several different other services could be provided, those cases need to be kept apart for unambiguous signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The binding indication is extended with a scope parameter that can indicate for a binding indication provided by the service consumer whether that binding indication relates to notifications to subscription events, to notification to other events, or to other services that the NF service consumer produces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The semantics of binding indications provided by the NF service consumer are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.0	Principles for Binding, Selection and Reselection

Binding can be used to indicate suitable target NF producer instance(s) for NF service instance selection, reselection and routing of subsequent requests associated with a specific NF producer resource (context) and NF service. This allows the NF producer to indicate that the NF consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set depending on local policies and other criteria (e.g. at what point it is in the middle of a certain procedure, considering performance aspects etc).

Binding can also be used by the NF consumer to indicate suitable NF consumer instance(s) for notification target instance selection, reselection and routing of subsequent notification requests associated with a specific notification subscription, and for providing binding indication for other service(s) that the NF consumer produces for the same data context and the NF service producer is subsequently likely to invoke.

The Binding indication is provided by the recipient of the messages the binding indication relates to. The Binding Indication contains the information in Table 6.3.1.0-1.

The Routing Binding Indication may be included in Request, Subsribe or Notification messages (see clause 7.1.2). It can be used in case of indirect communication by the SCP to route the message. The Routing Binding Indication also contains the information in Table 6.3.1.0-1.

NOTE x:	Subscription request messages can contain both the Binding Indication and the Routing Binding Indication.

The NF service producer may provide a Binding Indication to the NF service consumer as part of the Direct or Indirect Communication procedures, to be used in subsequent related service requests. The level of Binding Indication provided by the NF service producer to the NF consumer indicates if the resource in the NF service producer is either bound to NF service instance, NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. The Binding Indication may include NF Service Set ID, NF Set ID, NF instance ID, or NF service instance ID, for use by the NF consumer or SCP for NF Service Producer (re-)selection. If the resource is created in the NF Service Producer, the NF Service Producer provides resource information which includes the endpoint address of the NF service producer. For indirect communication, the NF service consumer also derives the Routing Binding Indication in Request or Subscribe message from that Binding Indication.

During explicit or implicit notification subscription, a Binding Indication may be provided by the NF service consumer to NF service producer. The NF service consumer may also provide a Binding Indication in replies to notification requests. The level of Binding Indication provided by the NF service consumer to the NF service provider indicates if the notification endpoint is either bound to NF service instance, NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. The Binding Indication may include, NF Set ID, NF instance ID, NF Service Set ID, NF service instance ID, and service name. The NF Service Set ID, NF service instance ID, and service name relate to the service of the NF service consumer that will handle the notification. The Binding Indication is used by the NF service producer as notification sender to (re )select the notification endpoint, i.e. the URI where the notification is to be sent. For indirect communication, the NF service producer also derives the Routing Binding Indication in Notification message from that Binding Indication. For subscription to notifications via another network function, a separate subscription for subscription related events can be provided; two binding indications for notifications are then provided and the Binding Indication for notifications to subscription related events shall include a scope parameter indicating "notification for subscription events".

If the NF consumer provides binding for other services in service requests, it shall indicate that the scope of the binding is for "other service" and indicate the related service name, in addition to the other parameters listed in Table 6.3.1.0-1. For NF Set or NF Instance level of binding, the binding indication for notifications and other services may be combined and the scope parameter shall then indicate all scenarios that the binding indication relates to. For NF Set or NF Instance level of binding for "other services", an endpoint address registered in the NRF at NF Profile level may be used to address the other services.

NOTE y:	Request messages can contain both the Binding Indications for other services and for notifications. and in addition the Routing Binding Indication.

Table 6.3.1.0-1 defines the selection and reselection behaviour of NF services consumers and SCPs depending on the binding indication provided by an NF service producer. The detailed procedures refer to clause 4.17.11 and 4.17.12 of TS 23.502 [3]

Table 6.3.1.0-1: Binding, selection and reselection

| Level of Binding indication | The NF Consumer / Notification sender / SCP selects | The NF Consumer / Notification sender / SCP can reselect e.g. when selected producer is not available | Binding information for selection and re-selection |
| --- | --- | --- | --- |
| NF Service Instance | The indicated NF Service Instance | An equivalent NF Service instance: - within the NF Service Set (if applicable) - within the NF instance - within the NF Set (if applicable) | NF Service Instance ID, NF Service Set ID, NF Instance ID, NF Set ID, Service name (NOTE 4), scope (NOTE a) |
| NF Service Set | Any NF Service instance within the indicated NF Service Set | Any NF Service instance within an equivalent NF Service Set within the NF Set (if applicable) (Note 2) | NF Service Set ID, NF Instance ID, NF Set ID, Service name (NOTE 4), scope (NOTE a) |
| NF Instance | Any equivalent NF Service instance within the NF instance. | Any equivalent NF Service instance within a different NF instance within the NF Set (if applicable) | NF Instance ID, NF Set ID, Service name (NOTE 4), scope (NOTE a) |
| NF Set | Any equivalent NF Service instance within the indicated NF Set | Any equivalent NF Service instance within the NF Set | NF Set ID, scope (NOTE a) |
| NOTE 1: if the Binding Indication is not available, the NF Consumer/SCP routes the service request to the target based on routing information available. NOTE 2: NF Service Sets in different NFs are considered equivalent if they include same type and variant (e.g. identical NF Service Set ID) of NF Services. NOTE 3: If a Routing Binding Indication is not available, the SCP routes the service request to the target based on available routing information. NOTE 4: The service name is only applicable if the Binding Indication relates to an notification target, or to other services that the consumer provides. For NF Instance and NF set level of binding, several service names for other services that the consumer provides can be supplied. NOTE a: The scope parameter is only applicable in a binding indication that the service consumer provides. The scope parameter can indicate that the binding relates to notifications to subscription events, to notification to other events, or to other services that the NF service consumer produces. For NF Instance and NF set level of binding, several scope values can be supplied in combination. The value "notification to other events" is the default if the scope parameter is omitted. |  |  |  |