3GPP TSG-SA WG2 Meeting #136-AH	S2-2001521

# Incheon, Korea, January 13-17 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2084 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Emergency services support by SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This addresses the following open item: #10 According to SA2, understanding is that emergency services is not supported by SNPN for any UE. However, TS 23.501 states the restriction only in the context of UE(s) in SNPN AM: -Emergency services are not supported in SNPN access mode. For UE(s) not in SNPN AM (i.e. regular UE(s) or pre-Rel-16 UE(s)), it is not stated that emergency services is not supported in the SNPN rather it is unclear in the spec whether emergency services must be offered by SNPN or not for UE(s) that are not in SNPN AM (i.e. if UE(s) in limited service state are camping in SNPN, should SNPN be able to offer emergency services?). It would be better to state whether emergency service is supported in SNPN or not from network perspective. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the SNPN does not support emergency services also from network perspective so it is clear that the SNPN need not offer emergency services support also for UE(s) in limited service state. Also, clarify roaming and HO between SNPNs are not supported in this release of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification leads to unclear requirement for RAN2 especially unclear whether UE(s) in limited service can be supported by SNPN or not. This can also lead to unclear expectation for implementation/deployments of SNPN network (i.e. features supported / not supported in the SNPN not clear). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.30	Support for non-public networks

### 5.30.1	General

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN may be deployed as:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public network integrated NPN, i.e. a non-public network deployed with the support of a PLMN.

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3, the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.8.2.1-1) for access to SNPN services via a PLMN (and vice versa) and the additional functionality covered in clause 5.30.2.

Interworking with EPS is not supported for SNPN. Also, emergency services are not supported by SNPN. Furthermore, roaming between SNPNs and handover between SNPNs are not supported.

Public network integrated NPNs can be enabled using network slicing (see Annex D).