SA WG2 Meeting #136-AH	S2-2001531

Incheon, South Korea, January 13 – 17, 2020	(revision of S2-2000761)(1366)

Source:	Samsung, NTT DOCOMO

Title:	Solution for SNPN and SP separation

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This solution addresses Key issue #1. especially, on how to exchange authentication signalling between the SNPN and the separate entity in PLMN.

1. Discussion

Per TR 23.700-07 describes the key issue #1 “Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN” including the following aspects:

-	How to exchange authentication signalling between the SNPN and the separate entity, including:

-	Authentication by the PLMN, based on PLMN identities and credentials, for access to the SNPN;

So, we are proposing a solution that UE authentication to access SNPN is performed based on out-of-SNPN entity (e.g. PLMN’s AUSF) using UE identity and credentials which the PLMN provides for SNPN use.

2. Proposal

It is proposed to add the following changes into TR 23.700-07.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| XX | X |  |  |  |  |  |

* * * * 2nd Change (new text) * * * *

6.X	Solution #XX: UE authentication for access to the SNPN using PLMN

6.X.1	Introduction

Editor's note:	This clause lists the key issue(s) addressed by this solution, and briefly the main principles of the solution.

In this solution, we address the key issue #1 ("Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN "). Especially the scenario that UE authentication to access SNPN is performed based on out-of-SNPN entity (e.g. PLMN’s AUSF) is considered.

6.X.2	Functional Description

Editor's note:	This clause further details the solution principles and any assumptions made.

We may assume that the SNPN is managed with same operator with the PLMN so that UE authentication can be done by the PLMN.

So UE is configured by UE identity and credentials which the PLMN provides for SNPN use.

- UE may discover & select the subscribing SNPN, based on the SNPN identification (i.e. PLMN ID and NID) broadcast from RAN, which is no difference from the current NW discovery/selection.

- During UE registration procedure to the SNPN, the AMF may decide to need UE authentication.

- Based on the UE identity (e.g. SUCI or 5G-GUTI) the UE sends, for UE authentication the AMF selects an AUSF in the PLMN which the UE identity indicates and additionally the SNPN’s NID can be used together to discover and select a proper AUSF.

- On successful UE authentication using the AUSF, the other remaining registration procedure is performed within the SNPN.

Figure 6..1-1: Standalone Non-public network supporting authentication using PLMN credentials

6.X.3	Procedures

Editor's note:	This clause describes procedures and information flows for the solution.

6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

AMF : it selects an AUSF in the PLMN for authorization and a UDM in SNPN for fetching UE SNPN subscription.

UDM : for SNPN UE authentication a UDM in PLMN is used, but for the SPN UE subscription a UDM in SNPN is used.

* * * * End of Changes * * * *